Boot Configuration v1.03 by Uwe Seimet

This  program  arrived  with  no  docs  and  in  German.  I  have

translated  everything  into  English but  cannot  guarantee  its

accuracy  as I am not a German speaker.  It is meant to run on  a

double  line or interlace mode but will operate in any 80  column

mode although the dialog boxes will appear to overlay each other.

The  Falcon (and possibly the TT) has an area of  battery  backed

RAM  used  for storing such things as the  country  and  keyboard

tables,  hertz rate and so on.  This program modifies the battery

backed  RAM so it should be used with caution.  It allows you  to

set  the country for all GEM/TOS menus and messages to  UK,  USA,

GERMAN, FRENCH, SPANISH or ITALIAN and the keyboard layout to any

one of the above as well as SWISS GERMAN and SWISS FRENCH.

Colours lets you pick from 2,  4,  16,  256 or 65536 colours  and

match  them  with  the column width (40 or 80)  of  your  choice.

Overscan  is  only possible on a TV or ordinary  colour  monitor.

Don't  try it on VGA!  Overscan removes the white borders in  any

resolution  and increases your resolution by a factor of  1.2  in

both  directions.   This  is  particularly  useful  for   editing

documents which are 80 characters wide, using a window based word

processor like First Word/Plus.

PAL or NTSC will set the display in colour to 50Hz (PAL) or  60Hz

(USA).  Double  line  on  is the native mode  for  VGA  monitors,

allowing  640x480  or 320x480.  Ordinary monitors can  achieve  a

maximum  depth  of  400 lines (close to VGA  standard)  by  using

interlace.  This  gives a very flickery display which is hard  on

the eyes.  However, it does allow you to run software which would

previously  only  run on high res on an ST.  You can  reduce  the

flicker  by  turning  down the brightness and  contrast  on  your

TV/monitor.

By selecting compatibility mode you will boot up in ST low or  ST

high  res,  depending  on  what type of  monitor  you  have.  The

compatibility  modes have been selected to allow you to run  some

incompatible programs,  they are not merely presets,  matching up

the correct colours and columns as those used in the  appropriate

ST resolution.

Date  and  Time are more or  less  self-explanatory.  These  will

affect how the computer expects to receive input for the  battery

backed  clock via the control panel or other  software.  It  also

affects  how  the  time  and  date  will  be  displayed,   unless

individual programs over-ride it.

Preferences  is a bit beyond me.  I assume it means that you  can

attach a Unix hard drive to your SCSI II port and read the  drive

without problems. I know that if Unix is selected, you can't read

an Atari formatted drive!  Don't worry,  it can be set back again

to TOS format by re-running the program.

Well,  this readme has gone on too long! Remember Floppyshop were

the  first PD/Shareware distributor in the UK to  release  Falcon

software and we will continue to support this great machine.

Steve Delaney, Floppyshop

April 1993