VKILLER                    Version 3.84                 April, 1991

This Archive contains the most recent version of VKILLER, the virus

detect-and-kill utility for the Atari ST.

The program works in medium or high resolution, and is completely

mouse/icon driven. The program may also be controlled by the keyboard.

In this document, all the keyboard commands are indicated as capital

letters, but that is not mandatory. Lower case letters will provide the

same functions. In most cases, the first letter of the label under an

icon is the key that will accomplish the same function of as a click on

the icon.

**** To Check Disks for Viruses ****

Click on the FLOPPY A icon, or press the "A" key, to check the disk in

drive A for a virus. Click on the FLOPPY B icon, or press the "B" key,

to check the disk in drive B for a virus. When you access a disk, the

program reads in the boot sector, both copies of the File Allocation

Table (FAT), the disk's root directory, and the first few data sectors.

The disk's boot sector will be checked to see if it is executable, and

if it matches any known patterns. If it is executable, a warning

message will be posted in the display window. If it matches the

patterns of some known boot sector utility, such as a resolution

setter, that fact will be illustrated by a box in the lower right

corner. If it matches the pattern of any known virus, that fact will be

indicated by a box in the lower right corner. To see more information

on any recognized virus, click on the box in the lower right corner, or

press the 'D' key for details.

**** Capturing Disk Data ****

Once you have accessed a disk to check it for a virus, you can write the

data from the disk into a file, print it, or show it on the screen.

To write the disk data into a file, click on the "FILE" icon, or press

the "F" key. A file selector will appear. Use it to designate the file

you wish to write. The resulting file is not executable, even if the

boot sector of the floppy was an executable one. It is a data file with

an image of the significant portions of the disk. It can be treated

like any data file, compressed by an archiving utility such as ARC or

LZH, copied, transmitted via electronic mail systems, printed, or

examined.

To print the data, click on the PRINT icon, or press the "P" key. An

alert box will appear. You may choose to print either the same data

that is available in the "SHOW" window, or only the data from the boot

sector. The amount of data captured from the disk varies, depending

upon the disk's configuration. For a typical ST disk, it will usually

be about 20 sectors, or slightly over 20,000 bytes. The boot sector

only is 512 bytes.

To show the data on the screen, click on the SHOW icon, or press the

"S" key. The window will expand to nearly the full screen, and display

all the data read from the disk. Use the window's scroll bar to move

back and forth through the data. Close the data window, by clicking on

the close box, to return to the main screen. Pressing any of the active

keyboard keys will also close the data window and return to the main

screen.

**** Disk Basics ****

The boot sector of a normal ST disk is 512 bytes. Only a small portion

of this, about 30 bytes, are required to provide data to the ST. Those

initial data bytes contain the disk's formatting characteristics,

telling the ST's operating system how many tracks the disk has, how

many sides are used, how many sectors are on each track, and how much

of the disk is being used for the directory and the File Allocation

Table. The rest of the boot sector is not used unless the disk is "self

booting". In this case, normally found only on games, the boot sector

is "executable", and the normally unused portion of the boot sector

contains a small program. This program will be executed automatically

when the ST is powered on, or reset, and the disk with the executable

boot sector is in drive A. This is true even if your ST is configured

to automatically boot up from a hard disk. The boot sector of a disk in

drive A will still be checked to see if it is executable, and will be

executed if it is. Executable boot sectors, therefore, are the primary

method used by viruses to spread, and be run by unsuspecting ST owners.

If the boot sector is not executable, the space after the configuration

data is unused. While whatever is there is not normally important,

there is one virus which utilizes an obscure system characteristic to

hide in the boot sector, and not make the disk "executable". To be

safe,  the unused portions of the boot sector of any non executable

disk should be set to zeroes. Some formatting programs do this, while

others do not. The ST desktop, for example, does not set the unused

portion of the boot sector to zero.

The boot sector also contains a serial number. That number is used by

the ST's operating system to determine when one disk has been removed,

and a different disk inserted. If a change is made between two disks

which have the same serial number, however, the ST does not believe

that there has been a disk change made. That usually results in the

destruction of the second disk, when the data written to it aligns with

the file structure of the removed disk. There are viruses which write

the same serial number to different disks, resulting in such

destruction. There is a similar problem using disks formatted by an

MS-DOS system. The serial number is not used by MS-DOS. Instead, MS-DOS

writes the version number in the space used for the serial number. This

results in all disks formatted on MS-DOS systems appearing to have the

same serial number, and becoming corrupted when they are used in an ST.

To warn of this possibility, VKILLER checks the serial number field. It

always displays the disk's serial number in the data window. If the

field contains printable characters, they are posted after the serial

number, in parenthesis. If you find the same serial number on more than

one disk, you can use VKILLER's Repair facility to renumber the disk,

without altering the disks contents in any other way.

The directory contains the names of any sub-directories (or "folders")

which are accessible from the main directory. Those sub-directories may

contain more subdirectories, and files. There may also be files in the

main directory. The directory size is specified when the disk is

formatted. The standard size for an ST directory is seven disk sectors,

large enough to hold 112 files or sub-directories. Few disks contain

that many files or folders in the main directory, so there is usually

some amount of unused space beyond the last entry in the directory.

There are viruses that will attempt to hide themselves in the end of

the directory. There is one that will place itself in the last two

sectors of the directory, whether that portion of the directory

contained entries or not. If that virus infects a disk which was using

the last two sectors of the dirctory, any file or directory which was

there will be lost.

All files, and sub-directories, rely upon the File Allocation Table to

be accessible. The File Allocation Table, or FAT, is a map of where

each file and sub-directory is recorded on the disk. The ST's operating

system relies upon the FAT to locate the proper portions of the disk in

order to read and write the sub-directories and files. The FAT,

therefore, is absolutely critical in using the disk. So critical, in

fact, that the ST's operating system normally writes two copies of the

FAT on the disk. That way, in case of an error in reading or writing

the first copy, the second copy may allow the data on the disk to be

recovered. The critical nature of the FAT also makes it a prime target

for virus attacks. Erasing the FAT usually results in the loss of all

files on a disk.

The size of the Directory and FATs may vary from disk to disk. The size

of the FAT must be large enough to record the layout of every portion

of the disk. The normal size of one copy of the FAT on an ST disk is

five sectors. This is more space than is necessary to record the disk's

layout. There are viruses that take advantage of this fact, and attempt

to hide themselves in the unused portion of the FATs.

This means that there are currently three places that viruses can hide

on a disk - in the unused portion of the boot sector, in the portion of

the reserved FAT space not needed for a particular disk, or beyond the

last entry in the directory. VKILLER checks disks for all these

possibilities. If it notes that there is something other than zeroes in

the unused portion of the boot sector, it displays a message reading

"Boot sector not zeroed". If the unused portions of the FAT or

directory are not zeroed, it displays a message reading "Extra sectors

not zeroed". These messages warn of the condition, but do not

necessarily mean that there is a problem.

One other value stored in the boot sector configuration data is

checked. There is a provision made for reserving additional disk

sectors, for some special use. These sectors are referred to as

"Hidden", since they are not accessible to any normal disk function.

There is no current use for hidden sectors on an ST disk, so the

presence of them is a condition which should cause concern. If VKILLER

detects hidden sectors on a disk, a warning message will appear in the

disk data window.

**** Cleaning up Disks ****

If the disk contains a virus, or garbage in the boot sector, you can

clean it up by clicking on the KILL icon, or pressing the "K" key. When

the program executes a "KILL", it writes zeroes into all the non

critical bytes in the boot sector. Whether the remaining unused

portions of the disk are altered depends upon an option set from the

"Options" menu, or by pressing the 'Z' key. The "Extra Zeroing" option

is on when the program begins executing. If this option remains on,

then executing a Kill on a disk will also cause the unused portions of

the FAT, and the unused portion of the directory, to be set to zero. If

the "Extra Zeroing" option is off, only the unused portion of the boot

sector is set to zero. When "Extra Zeroing" is selected from the menu,

or the 'Z' key is pressed, a map of the current disk is displayed,

showing the status and use of each sector in the boot, FAT, and

directory portions of the disk.

No other portion of the disk is altered, and any files on the disk are

left intact. The resulting boot sector provides a disk readable by both

ST's and MS-DOS systems. It is not necessary to display a disk's data

before executing a kill on it. You can insert a disk and press "K", or

click on the Kill icon, immediately. The disk's boot sector will be

read, the critical portions preserved, and the non critical portions

zeroed out.

If you wish to install a "Guard" boot sector on a disk, click on the

guard icon, or press the "G" key. A dialog box will appear, offering a

choice of two types of guard boot sectors.

The first guard boot sector is the "Display" type. It contains a simple

program which will display the message "Virus free disk" when the

system is powered up or reset with that disk in drive A. Once the

message has been displayed, the program returns to the operating system

to continue with the power up sequence. It does not remain in memory.

If you reset the system with that disk in drive A, and the message does

not appear, you should immediately check that disk for a virus. The

only reason why the message would not appear is if the boot sector has

been altered, possibly by a spreading virus. This guard boot sector is

an adaptation of one originally written by Mark S. Powell.

The second guard boot sector is the monitoring type. This one also

displays a message at power up and reset. Unlike the display version,

however, this guard remains active in memory until the system is reset

again. Of course, if the disk in drive A at the next reset contains the

guard boot, it will be reloaded again. Otherwise, it is removed from

the system.

While the monitor is active, it checks the boot sector of every disk

that is accessed by the ST. If it detects an executable boot sector on

any disk, it will flash the screen colors, and sound a warning tone. If

the disk just accessed was not one that should have an executable boot

sector, you should become suspicious. The monitor will not issue a

warning for disks which contain copies of itself.

Vkiller now contains a feature for repairing the boot sector of a

damaged disk. Activate it by clicking on the repair icon, or pressing

the "R" key. It will not re-create executable boot sectors, such as

those required for self booting games. When a disk boot sector is

damaged, the disk will be deemed unreadable by the ST's operating

system. If only the boot sector is damaged, repairing it may allow the

data on the disk to be recovered.

When you activate the repair function, a dialog box appears. It

contains all the data that can vary in a disk's boot sector. The

individual entries are each editable. They will originally contain

whatever value was read from the disk. This may not be reasonable or

valid if the boot sector was corrupted. The actual data that is in the

boot sector is a bit more obscure than what the dialog presents, but

the needed data can be calculated from what is presented in the dialog.

OS ID: This is an ASCII data string. It is designed to

provide the identity of the operating system under which

the disk was formatted. It can contain anything. It is

not used by the ST operating system in the current version,

but is supplied for MS-DOS compatibility.

Serial Number: This number is used to tell disks from each

other. The number should be different on every disk. This

box will originally contain the number that came from the disk.

You can enter any value you wish, if you so desire. The

legal range of entries is from 0 - 16,777,215. It should

be stated here that the three bytes in which this number is

stored are also used for the operating system ID on MS-DOS

systems, and will usually have the version number of the

formatter. If you regularly move disks back and forth with

MS-DOS systems, you may encounter problems using them in your

ST, since many will appear have the same serial numbers.

You shoud, therefore, avoid the numbers which correspond

to the MS-DOS versions:

1.0 - 3,223,088 through 1.9 - 3,223,097

2.0 - 3,288,624 through 2.9 - 3,288,633

3.0 - 3,354,160 through 3.9 - 3,354,169

4.0 - 3,419,696 through 4.9 - 3,419,705

If VKILLER notices that all three characters in the disk

serial number are printable ASCII characters, it will display

the three characters in parenthesis in the disk data window.

Format ID - Also for MS-DOS compatibility, and not used

by the ST. The normal ID value is F9.

Reserved Sectors - The number of sectors at the beginning

of the disk which are not used for the FATs, Directory,

or data. Unless you are attempting to something very

unusual, this should always be 1 (for the boot sector).

Hidden Sectors - Sectors at the beginning of the disk

which should not be accessed. This should be zero.

Sides on Disk - Either 1 or 2. If you come up with a

three sided disk, I'd be very interested in seeing it.

Tracks on Disk - The standard format for an ST is 80.

Extended formatters may provide 81 or 82, if your drive

can step in that far. Some 5.25" disk drives have 40 tracks.

Sectors per Track - The standard for the ST (and MS-DOS)

is 9. Some extended formatters, including "TWISTER",

use 10, while others will go to 11. The ST's desktop

formatter always uses 9.

Sectors per FAT - The FAT is the File Allocation Table.

It is a map of how the folders and files are positioned

on the disk. The ST standard is 5. This is more than

adequate, since the capacity of the disk only requires

3. A disk formatted in an MS-DOS system will have 3.

When viewed as data, a FAT looks like a bunch of garbage.

There will always be two copies of it, at the start of a

disk. You can use the "Show" feature of Vkiller to look

at the disk. If sectors 1 and 4 look the same, and the

directory starts at sector 7, the disk has a 3 sector FAT.

If sectors 1 and 6 look the same, and the directory starts

in sector 11, the disk has a 5 sector FAT.

Directory Sectors - The number of disk sectors provided

for the disk directory. The ST standard is 7.

If you accessed the repair function only to change the serial number on

the disk, click on the "Renumber" button. A random serial number will

be generated, the disk boot sector will be updated, and the program

will return to the data window. If you want to assign a  specific

serial number, move the cursor to the serial number entry, type in a

number in the range 0 - 16,777,215, and select the "Rewrite" button.

If the data in the dialog appears to be nonsense, the disk's boot

sector has been corrupted. This may be due to a wide range of reasons,

not just a virus. To set the values to the ST standards, you can enter

the appropriate values for each item, or select the "Set Defaults"

button. It will set the normal values for most of the entries, and

generate a random serial number. You should, however, insure that the

number of tracks, sectors, and sides on the disk are correct for that

particular disk. If you aren't certain, you can use the "Scan Disk"

function.

When you select the scan disk function, a dialog with a number of

selectable boxes appears. To repair a damaged disk, the program must

know the number of tracks, sectors per track, and sides on the disk. If

you know any of those facts, enter them in the dialog by clicking on

the appropriate boxes. If you don't know, leave the boxes unselected.

The program will attempt to determine the disk's configuration by

reading different areas, to establish any of the entries that you do

not provide. You can also have a set of default values inserted by

clicking on the "Default" button.

Since the number of tracks on a disk may vary, the program will attempt

to determine how many tracks were originally on the disk by stepping

the disk head in and reading data, until it reaches an unreadable area.

It will not continue stepping in past an unreadable track, but it will

continue stepping in until it does reach an unreadable track. This, of

course, will be one track further than the the disk's formatting

originally extended. While it is extremely unlikely, it is possible

that repeated execution of this stepping in operation, which could

extend beyond the range of the drive, could cause the disk's head to

become mis-aligned. It has not occurred through weeks of testing, and

would probably require thousands of such operations, but it is

possible. If you wish to prevent the head from stepping in past some

specific track, click on the last track you wish the disk to access,

and select the "Limit" box. The head will not be stepped in beyond the

track you select, even if that track does contain readable data. Of

course, if an unreadable track is detected before that limit is

reached, no further stepping will occur.

After all the physical configuration parameters have been entered or

determined, the original dialog box will re-appear. The values

determined by scanning the disk will have been updated. You may edit

them again, if you wish, before writing the disk's boot sector. After

the disk has been updated, the program returns to the familiar disk

data window.

Since this boot sector rebuilding function starts with the data

currently on the disk, it can also be used as a means of altering the

data on a non-infected disk, such as setting serial numbers.

The Menu offers two features under the "Options" title. The first

option, "Quiet", will turn off the warning siren that sounds when a

virus is detected. Clicking on "Quiet" again will turn the siren back

on.

Since some viruses check system data, such as ROM dates, to determine

if they can execute, a second "Option" is provided. Clicking on "System

Info" will cause the data window to display information about the

system parameters.

Exit the program by clicking on the "QUIT" icon, or pressing the "Q"

key.

When the program detects a virus present on a disk, it will sound a

warning tone, and place a new button on the desktop. To see the details

about the particular virus located, click on the new "Known Virus" box,

or press the "D" key. If you find a lot of disks infected by a virus,

and want to stop the warning tone from sounding on each disk,  select

the "QUIET" item from the "OPTIONS" menu.

If the disk contains an executable boot sector, but one that is

recognized as a standard system boot, it will be identified as a

"System Disk".

**** General Hints ****

There are two real problems to keep in mind when dealing with viruses,

and disk boot sectors. The first is that not all executable boot

sectors are viruses. There are times when a boot sector is supposed to

be executable. Executing a KILL on such a boot sector will destroy

whatever code was in the boot sector, and may render the software on

the disk useless. In order to determine if the software on a disk

requires an executable boot sector, you may execute a "Test Kill" from

the "Options" menu, or pressing the 'T' key. This will render the disk

boot sector un-executable, but in a manner that may be restored back to

the original executable status. A dialog box will appear, explaining

the test kill procedure, and offering the buttons "Test Kill",

"Cancel", and "Restore Boot". Select "Test Kill" to determine if a disk

which bears an executable boot sector will function normally without an

executable boot sector. The disk will be altered to non-executable

status. Exit VKILLER, and attempt to use the disk as you had

previously. If it still works properly, it did not need the executable

boot sector. You should rerun VKILLER, and do a  normal "Kill" to erase

whatever was in the boot sector. If the disk will not function properly

after the test kill, rerun VKILLER immediately and execute the "Restore

Boot" function. This will return the boot sector to the prior status.

You should record the fact that that particular disk must contain an

executable boot sector, to avoid acidentally removing it at a later

time.

Generally speaking, if the proper use of the software on the disk

required you to reset your ST, or power it off and on to start the

program, then the boot sector was supposed to be executable, and you

should not execute a virus kill on it. Any program which can be

executed by clicking on it does not require an executable boot sector.

Consequently, disks used to store such programs should not contain

executable boot sectors.

The second problem is attempting to use VKILLER in a system which has

been infected by a virus, and the virus is executing (attempting to

spread) while VKILLER is attempting to clean it off disks. With all

known ST viruses as of this release (January, 1991), VKILLER will

detect this situation. It will provide instructions on the screen,

informing you that there is an active virus in the system at the time,

the exact steps on how to get rid of it, and how to start cleaning up

your disks.

This version of VKILLER can recognize and eliminate over 30 different

ST viruses. This is possible only because people who have encountered

viruses that earlier versions of the program did not recognize sent me

copies of the new viruses. If you encounter a virus that the program

does recognize, there is no need to contact me. Just eliminate the

virus, and let anyone else you have given a disk to know that their

system may be infected. Give them a copy of this program if you wish,

it is free to anyone who would like it.

If, however, you encounter executable boot sectors spreading through

your library, and this version of VKILLER can't identify it, please

contact me at any of the addresses below. Keep one infected disk, and

either send me the disk, or the file generated by VKILLER's file

function. Use the Kill function to clean up the rest of the disks in

your library.

As of this writing, I am investigating about 40 disks each month for

new viruses. If you mail me a disk, or a request for a disk copy of the

latest version, please include a stamped, self addressed return mailer.

Please be patient in expecting a response.

VKILLER does not require any "license", "registration", or "shareware"

contributions. Of course, all such contributions are gratefully

accepted, but none are solicited. Circulate the program in any manner

you wish. It may be copied, and distributed freely, but it may not be

sold. Reasonable (and hopefully modest) charges for media, copying, or

downloading are acceptable.

George R. Woodside

Voice:         (818) 348-9174

Compuserve:    76537,1342

GEnie:         G.WOODSIDE

USENET:        woodside@ttidca.com

or:        ..!{philabs|csun|psivax}!ttidca!woodside

US MAIL:       5219 San Felicaino Drive

Woodland Hills, CA  91364  USA