DT - Disk Test Program

GENERAL

The DT program is used to test TOS disk partitions for sector errors

and optionally map out bad sectors.  It is a GEM program and can be

run directly from the GEM Desktop in medium or high resolution. It

should not be used in low resolution because some of the dialogs

are dificult to read in this setting.

WHAT YOU NEED TO KNOW BEFORE YOU USE DT

DT test reads every sector on a TOS formatted disk partition and

reports errors to the operator.   First it test reads the system

area of the disk (boot, FAT, and root directory).   If this area

contains any disk errors, there is no way to fix it except to

re-format the disk (this will be explained below).  Next, the

program test reads all the data clusters of the disk.  If a data

cluster contains a bad sector, there are three possibilities:

1) The cluster is free.  It isn't currently used by any files or

directories on the disk.   This is the best case.  The sector

can be safely marked bad so that it won't be used for data.

2) The cluster is currently storing file or directory data. The

file or directory is in jeopardy.   In fact, the file is not

usable by TOS and continued use of the disk could result in

severe file damage.   If the cluster is in the middle of the

file then the file will be truncated, if you tell DT to mark the

cluster bad.   The best way to handle this case is to use DT

to mark the cluster bad and then use SENTRY to locate the

affected files.   DO NOT USE SENTRY TO REPAIR THE DAMAGE YET.

First exit SENTRY and delete the files in jeopardy, then go

back and use DT again to re-test the disk.   Finally, use

SENTRY to repair the disk.  SENTRY should report the correct

number of unusable clusters. If it does not, repeat the

procedure until it does.   This is a very dangerous case and

extreme care must be exercised to remedy it.

3) The cluster is already marked bad.  In this case the error

is counted but no action is taken, because the cluster is

already safe from use.

TOS has no way to map out bad sectors in the FAT or root

directory.   If this area contains disk errors, it is necessary

to re-format the disk.  Continued use of a disk with bad

sectors in the system area will cause un-recoverable data loss.

HARD DISK INFORMATION

The DT program should be considered a temporary solution to

hard disk errors.  It is useful for verifying that the disk

is working reliably, but it should not be used as the final

method of mapping out bad hard disk sectors.   It will allow

you to keep using the hard disk until you are ready to use

your hard disk manufacturers low-level formatting utilities.

The low-level format tools are the correct method of mapping

out bad hard disk sectors.  The DT program should be used

to test hard disk partions, but bad sectors should be mapped

out with your hard disk manufacturers utilities.

USING THE DT PROGRAM

Open the DT.PRG Icon to start the program.   The DT.RSC file must be

present in the same folder as the DT.PRG file.   When the program

starts, a dialog is displayed.   From this dialog, select the TOS

disk that you want to test.   It will test one partition at a time

it can be a floppy disk or a hard disk partition.  It can also test

a RAM disk, but this isn't too useful, since it's unlikely you will

ever get a disk error from a RAM disk!

After selecting the drive you want to test, chose one of the four

options at the bottom of the dialog:  Test Once, Test Forever, Info,

or Quit.  These four options are described below.

TEST ONCE

This option test reads the selected disk, as described above.  Any

errors are reported. When the test is complete, the program returns

to the main startup dialog.  You may terminate the test by clicking

the mouse at any time during the test.  A final error count is

displayed.

TEST FOREVER

This option repeats the above tests forever, until you click the

mouse and request the program to stop.

INFO

This option displays informationm about the DT program.

QUIT

This optuion returns to the Desktop (or whatever program you used

to start DT).

DISCLAIMER

Beckemeyer Development makes no warranties either expressed or

implied, with respect to the software described in this document,

its quality , performance, merchantability, or fitness for any

particular purpose.  The software is licensed "as is".  The

entire risk as to the quality and performance of the software

is with the buyer.

In no event will Beckemeyer Development be liable for direct,

incidental, or consquential damages resulting from the use of

this software.

USE AT YOUR OWN RISK!