******************

*  TLC FORMATTER *

*   Version 2.00 *

******************

*** Version 2.00 additions:

There are 2 new buttons in the formatting box. Verify will check each track

after it's formatted to ensure there are no weak sectors on the disk. This

takes longer but if you really want to make sure your disks are okay, use

it. Alerts simply allows you to turn the alert boxes on or off. If you turn

them off, be sure you don't format a good disk. As is the popular trend,

this now formats backwards (that is, from track 80 to track 1). If you abort

before it's finished, any files near the front of the disk will still be

there.

I. USE

This program is very simple to use, it runs as either an accessory or a

program by changing the extender (i.e. '.ACC' for an accessory and '.PRG'

for a program) and runs in Medium or High resolution. The main screen shows

all the available options, you may select either A or B to format, 9 or 10

sectors per track and between 80 and 82 tracks per disk. The 'FaST' format

option does NOT make the formatting process any faster but rather formats a

disk in a way that makes it read and write even faster than disks formatted

with Twister or with 'skewed' sectors like DC Format uses. The fast option

only works with 9 sectors per track. To enter a label for the disk, simply

type it on the main screen. If an error is encountered during the formatting

process, the applicable track and side number will be shown and you will

have a choice as to continue attempting the format or to abort the

operation. For info on the program, click on the 'TLC' Icon. The drive

button for drive B: will only be available if you have 2 floppies attached.

If you do have 2 drives, while the format is in progress you may press

either A or B on your keyboard to signify that that disk should be

immediately formatted when the first is done. The program uses ALL the same

parameters, single/double sided, number of tracks, sectors and so forth. It

even assigns the same volume name to both disks. You may continue drive

switching as long as you like. The formatting process can be aborted at any

time by pressing the Esc key.

II. EXPLANATION

The 'FaST' option isn't really difficult to implement at all. When

using the XBIOS 10 call to format, simply set the interleave value to 11 as

opposed to the usual value of 1. Perhaps I can find how to make it work with

other than 9 sectors per track to increase storage space on your disks.

Waiting for files to load from floppies is really time consuming, especially

long files like digitized sounds or long programs like PageSTream or Touch

Up. The FaST option makes a noticeable difference in these cases.

III. SHAREWARE

You have no idea how few people donate to us 'home brewed' programmers.

It certainly isn't enough to justify all this equipment! (Not to the boss

anyway). With enough support, programs such as this will continue to grow. I

will notify all contributors in the event of any major additions or bug

fixes. A $15 contribution gets you the latest version of not only this

program, but at least 6 other 'TLC' programs all personalized for you.

I hope you enjoy using this program and I look forward to

hearing from you soon...

American address             German address

MSgt Tom Hayslett            Tom Hayslett

HQ USAFE/FMF                 In den Langwiesen 22, Apt 1

PSC 2, Box 6195              6790 Landstuhl

APO AE  09012                GERMANY