The incredible Help Program for Blue Boxing

---------------------------------------------

Presented by Keepermann

also known as the "THE VIRUS"

You can read downside the using of our little Program

NOTE:

a Modem link virus in the original program is removed...

please check your "libs/icon.library" if the size is 6680 bytes,

your library is infected..

What makes this a virus?????

your modem connection to bbs's would end up with a lot of noise, and

crap on the line(LOOKS LIKE CRAP), more about the virus is available

in the original program for programmers!

Skid Row

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Better Homes and Blue Boxing

Part I

Theory of Operation

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

To quote Karu Bluel Marx, blue boxing has always been the most able form of

phreaking. As opposed to such things as using an MCI code to make a free

fone call, which is merely mindless pseudo-phreaking, blue boxing is

actual interaction with the Bell System toll network. It is likewise

advisable to be more cautious when blue boxing, but the careful phreak

will not be caught, regardless of what type of switching system he is under.

In this part, I will explain how and why blue boxing works, as well as

where. Ind  h later parts, I will give more practical information for blue

boxing and routing information.

To begin with, blue boxing is simply communicating with trunks. Trunks must

not be confused with subscriber lines (or "customer loops") which are

standard telefone lines. Trunks are those lines that connect central

offices. Now, when trunks are not in use (i.e., idle or "on-hook" state)

they have 2600Hz applied to them. If they are two-way trunks, there is

2600Hz in both directions. When a trunk IS in use (busy or "off-hook"

state"), the 2600Hz is removed from the side that is off-hook. The 2600Hz

is therefore known as a supervisory signal, because it indicates the

status of a trunk; on hook (tone) or off-hook (no tone). Note also that

2600Hz denoted SF (single frequency) signalling and is "in-band." This is

very important. "In-band" means that is is within the band of frequencies

that may be transmitted over normal telefone lines. Other SF signals, such

as 3700Hz are used also. However, they cannot be carried over the telefone

network normally (they are "out-of- band") and are therefore not able to

be taken advantage of as 2600Hz is.

Back to trunks. Let's take a hypothetical phone call. You pick up

your fone and dial 1+806-258-1234 (your good friend in Armarillo,

Texas). For ease, we'll assume that you are on #5 Crossbar switching and

not in the 806 area. Your central office (CO) would recognize that

806 is a foreign NPA, so it would route the call to the toll centre

that serves you. [For the sake of accuracy here, and for the more

experienced readers, note that the CO in question is a class 5 with

LAMA that uses out-of-band SF supervisory signalling]. Depending

on where you are in the country, the call would leave yq  iour toll centre

(on more trunks) to another toll centre, or office of higher "rank".

Then it would be routed to central office 806-258 eventually and the

call would be completed. Illustration:

A---CO1-------TC1------TC2----CO2----B

A=you

CO1=your central office

TC1=your toll office.

TC2=toll office in Amarillo.

CO2=806-258 central office.

B=your friend (806-258-1234)

In this situation it would be realistic to say that CO2 uses SF

in-band (2600Hz) signalling, while all the others use out-of-band

signalling (3700Hz). If you don't understand this, don't worry too much.

I am pointing this out merely for the eake of accuracy. The point is that

while you are connected to 806-258- 1234, all those trunks from YOUR

central office (CO1) to the 806-258 central office (CO2) do *NOT* have

2600Hz on them, indicating to the Bell equipment that a call is in

progress and the trunks are in use.

Now let's say you're tired of talking to your friend in Amarillo

(806-258-1234) so you send 2600Hz down the line. This tone travels down

the line to your friend's central office (CO2) where it is detected.

However, a that CO thinks that the 2600Hz is originating from Bell

equipment, indicating to it that you've hung up, and thus the trunks

are once again idle (with 2600Hz present on them). But actually, you

have not hung up, you have fooled the equipment at your friend's CO into

thinking you have. Thus,it disconnects him and resets the equipment to

prepare for the next call. All this happens very quickly (300-800ms for

step-by-step equipment and 150-400ms for other equipment).

When you stop sending 2600Hz (after about a second), the equipment thinks

that another call is coming towards it (e.g. it thinks the far end has

come "off-hook" since the tone has stopped. It could be thought of as a

toggle switch: tone --> on hook, no tone -->off hook. Now that you've

stopped sending 2600Hz, several things happen:

1) A trunk is seized.

2) A "wink" is sent to the CALLING end from the CALLED end indicating that

the CALLED end (trunk) is not ready to receive digits yet.

3) A register is found and attached to the CALLED end of the trunk within

about two seconds (max).

4) A start-dial signal is sent to the CALLING end from the CALLED end

indicating that the CALLED end is ready to receive digits.

Now, all of this is pretty much transparent to the blue boxer. All he

really hears when these four things happen is a <beep><kerchunk>. So,

seizure of a trunk would go something like this:

1> Send a 2600Hz

2> Terminate 2600Hz after 1-2 secs.

3> [beep][kerchunk]

Once this happens, you are connected to a tandem that is ready to obey your

every command. The next step is to send signalling information in order

to place your call. For this you must simulate the signalling used by

operators and automatic toll-dialing equipment for use on trunks. There

are mainly two systems, DP and MF. However, DP went out with the dinosaur

, so I'll only discuss MF signalling. MF (multi-frequency) signalling is the

signalling used by the majority of the inter- and intra-lata network. It is

aO  also used in international dialing known as the CCITT no.5 system.

MF signalling consists of 7 frequen- cies, beginning with 700Hz and

separated by 200Hz. A different set of two of the 7 frequencies represent the

digits 0 thru 9, plus an additional 5 special keys. The frequencies and uses

are as follows:

Frequencies (Hz)  Domestic    Int'l

--------------------------------------

700+900             1          1

700+1100            2          2

900+1100            3          3

700+1300            4          4

900+1300            5          5

1100+1300            6          6

700+1500            7          7

900+1500            8          8

1100+1500            9          9

1300+1500            0          0

700+1700           SM         Code 11

900+1700           STp        Code 12

1100+1700           KP         KP1

1300+1700           ST2p       KP2

1500+1700           ST         ST

The timing of all the MF signals is a nominal 60ms, except for KP, which

should have a duration of 100ms. There should also be a 60ms silent period

between digits. This is very flexible, however, and most Bell equipment will

accept outrageous timings.

In addition to the standard uses listed above, MF pulsing also has

expanded uses known as "expanded inband signalling" that include such

things as coin collect, coin return, ringback, operator attached, and

operator released. KP2, code 11, and code 12 and the ST (STart "primes")

all have special uses which will be mentioned only briefly here.

To complete a call using a blue box, once seizure of a trunk has been

accomplished by sending 2600Hz and pausing for the <beep><kerchunk>, one

must first send a KP. This readies the register for the digits that follow.

For a standard domestic call, the KP would be followed by either 7 digits

(if the call were in the same NPA as the seized trunk) or 10 digits (if the

call were not in the same NPA as the seized trunk). [Exactly like dialing a

normal fone call. Following either the KP and 7 or 10 digits, a STart is

sent to signify that no more digits follow. Example of a complete call:

1> Dial 1-806-258-1234

2> wait for a call-progress indication (such as ring, busy,

recording, etc.)

3> Send 2600Hz for about 1 second.

4> Wait for about 2 seconds while a trunk is seized.

5> Send KP+305+994+9966+ST

The call will then connect if every- thing was done properly. Note that if

a call to an 806 number were being placed in the same situation, the area

code would be omitted and only KP+ seven digits+ST would be sent.

Code 11 and code 12 are used in international calling to request

certain types of operators. KP2 is used in international calling to route

a call other than by way of the normal route, whether for economic or

equipment reasons.

STp, ST2p, and ST3p (prime, two prime, and three prime) are used in

TSPS signalling to indicate calling type of call (such as coin-direct

dialed).

This has been Part I of Better Homes and Blue Boxing. I hope you

enjoyed and learned from it. If you have any questions, comments, threats

or insults, please fell free to drop me a line. If you have noticed any

errors in this text (yes, it does happen), please let me know and

perhaps a correction will be in order. Part II will deal mainly with more

advanced principles of blue boxing, as well as routings and operators.

Note 1: other highly trunkable areas include: 816,305,813,609,205.

I personally have excellent luck boxing off of 609-953-0000. Try that

if you have any trouble.

Better Homes and Blue Boxing

Part II

Practical Applications

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

(It is assumed that the reader has read and understood Part i of this series).

The essential purpose of blue boxing in the beginning was merely to receive

toll services free of charge. Though this can still be done, blue boxing has

essentially outlived its usefulness in this area. Modern day "extenders" and

long distance services provide a safer and easier way to make free fone calls.

However, you can do things with a blue box that just can't be done with any-

thing else. For ordinary toll-fraud, a blue box is impractical for the

following reasons:

1. Clumsy equipment required (blue box or equivalent)

2. Most boxed calls must be made through an extender. Not for

safety reasons, but for reasons I'll explain later.

3. Connections are often sacrificed because considerable distances

must be dialed to cross a seizable trunk, in addition to

awkward routing.

As stated in reason #2, boxed calls are usually made through an extender.

This is for billing reasons. If you recall from Part i, 2600Hz is used as a

"supervisory" signal. That is, it signals the status of a trunk--

"on-hook" or "off-hook." When you seize a trunk (by briefly sen+  ding

2600Hz), your end (the CALLING end) goes on hook for the duration of the

2600Hz and then goes off-hook once again when the 2600Hz is terminated.

The CALLED end recognizes that a call is on the way and attaches a register,

which inerprets the diE  gits which are to be sent. Now, understand that even

though your end has come off-hook (no 2600Hz present), the other end is

till on-hook. You may wonder then, why, if the other end (the CALLED end)

is still on-hook, there is no 2600Hz coming the other way on the trunk,

when there should be. This is correct. 2600Hz *IS* present on the trunk when

you seize it and afterwards, but you cannot hear it because of a Band

Elimination Filter (BEF) at your central office.

Back to the problem. Remember that when you seize a trunk, 2600Hz is

indeed coming the other way on the trunk because the CALLED end is still

on-hook, but you don't actually hear it because of a filter. However, the

Bell equipment knows it's there (they can "hear" it). The presence of the

2600Hz is telling the billing equip- ment that your call has not yet been

completed (i.e., the CALLED end is still on-hook). When finally you do

connect with your boxed call, the 2600Hz from the called end terminates.

This tells the billing equipment that someone picked up the fone at the

CALLED end and you should begin to be billed. So you do start to get billed,

but for the call to the trunk, NOT the boxed call. Your billing equipment

thinks that you've connected with the number you used to seize the trunk.

Illustration:

1. You call 1+806-258-2222

(directly)

2. Status of trunks:

<---- you ------------------------------->

(You)                    806-258-2222

No 2600Hz-------> <------------2600Hz

When you seize a trunk (before the number you called answers) there is

no affect on your billing equipment. It simply thinks that you're still

waiting for the call to complete (the CALLED end is still on-hook; it

is ringing, busy, going to recorder or intercepts operator.

Now, let's say that you've sezied a trunk (806-258-2222) and for example,

KP+314+949+1705+ST. The call is routed from the tandem you seized to:

314-949-1705.

Illustration:

<------------------>O<--------------->

(You)              806         314-949

tandem

No 2600Hz----------> <----------2600Hz

Note that the entire path towards the right (the CALLED end) has no

2600Hz present and is therefore "off- hook." The entire path towards the left

(the CALLING end) does have 2600Hz present on it, indicating that the

CALLED end has not picked up (or come "off-hook"). When 314-949-1705 answers,

"answer supervision" is given and the 2600Hz towards the left (the CALLING

end) terminates. This tells your billing !  nequipment, which thinks that

you're still waiting to be connected with 806-258-2222, that you've

finally connected. Billing then begins to 806-258-2222. Not exactly an

auspicious beginning for an aspiring young phone phreak.

To avoid this, several actions may be taken. As previously mentioned,

one may avoid being charged for the number called to seize a trunk by

using an extender (in which case the extender will get billed). In some

areas, boxing may be accomplished using an 800 number, generally in the

format of 800-858-xxxx (many Amarillo numbers) or 800-NN2-xxxx (special

intra-state class in-WATS numbers). However, boxing off of 800 numbers is

impossible in many areas. In my area, Denver, I am served by #1A ESS and it

is impossible for me to box off of any 800 number.

Years ago, in the early days of blue boxing (before my time), phreaks often

used directory assistance to box off of because they were "free" long

distance calls. However, because of competetive long distance con  mpanies,

directory assistance surcharges are now $0.50 in many areas. It is

additionally advised that directory assistance numbers not be used to box

from because of the following:

Average DA calls last under 2 minutes. When you box a call, chances

are that it will last considerably longer. Thus, the Bell billing equip-

ment will make a note of calls to directory assistance that last a long

time. A call to a directory assistant lasting for 4 hours and 17 minutes

may appear somewhat suspicious.

Although the date, time, and length of a DA call do not appear on the bill,

it is recorded on AMA tape and will trip a strouble report if it were to

last too long. This is how most phreaks were discovered in the old

days. Also, sometimes too many calls lasting too long to one 800 number

may raise a few eyebrows at the local security office.

Assuming you can complete a blue box call, the following are listed routings

for various Bell internal operators. These are in the format of KP+NPA+

special routing+1X1+ST, which I will explain later. The 1X1 is the actual

operator routing, and NPA and NPA+ special routing are used for out-of-

area code calls and out-of-area code calls requiring special routing,

respectively.

KP+101+ST ...... a toll test board

KP+121+ST ...... inward op

KP+131+ST ...... directory assistance

KP+141+ST ...... was rate & route. Now only works in 312, 815, 717,

and a few others. It has been replaced with a univer-

sal rate & route number, 800+141+1212.

KP+151+ST ...... overseas completion operator (inbound). Works

only in certain NPAs, such as 303.

KP+181+ST ...... in some areas, toll station for small towns

Thus, if you seize a trunk 806 NPA and wanted an inward (in 806), then you

would dial KP+121+ST. If you wanted a 312 inward and were dialing on an 806

trunk, an area code would be required. Thus, you would dial KP+312+121+ST.

Finally, some places in the network require special routing, in addition to

an area code. An example is Franklin Park, Ill. It requires a special

routing of 032. For this, you would dial KP+312+032+121+ST for a Franklin

Park inward operator.

Special routings are in the format of 0XX. They are used primarily for

load balance, so that traffic flow may be evenly distributed. About half

of the exchanges in the network require special routing. Note that

special routings are NEVER EVER EVER used to dial normal telephone numbers,

only operators.

Operator functions:

TOLL TEST BOARD- Generally a cordboard position that assists in trunk testing.

They are not used by operators, only switchmen.

INWARD- Assists the normal TSPS (0+) operator in completing calls out of

the TSPS's area. Also, inwards perform emergency inerrupts when the number to

be interrupted is out of the area code of the original (TSPS) operator. For

example, a 303 operator has a customer that needs an emergency interrupt on

215-647-6969. The 303 operator gets the routing for the inward that covers

215-647, since she cannot do the interrupt herself. The routing is

found to be only 215+ (no special routing required). So, the 303 operator

keys KP+215+121+ST. An inward answers and the 303 says to her, "Inward, this

is Denver. I need an emergency interrupt on 215-647-6969. My

customer's name is Mark Tabas." The inward will then do the interrupt (off

the line, of course). If the number to be interrupted had required special

routing, such as, say, 312-456-1234 (spec routing 032), then the 303

operator would dial KP+312+032+121+ST for the inward to do that interrupt.

DIRECTORY ASSISTANCE- These are the normal NPA+555+1212 operators that

assist customers with obtaining telefone directory listings. Not much

toll-fraud potential here, except maybe $0.50.

RATE AND ROUTE- These operators are reached by dialing KP+800+141+1212+ST.

They assist normal (TSPS) operators with rates and routings (thus the

name). The only uses I typically have for them are the following:

1. Routing information. In the above example, when the 303 operator needed

to dial an inward that served 215-647, she needed to know if any special

routing was required and, if so, what it was. Assuming he would use rate

and route, she would dial them and say nicely, "Operator's route, please, for

215-647." Rate & route would respond with "215 plus." This means that the

operator would dial KP+215+121+ST to reach the inward that serves 215-647.

If there were special routing required, such as in 312-456, rate & route would

respond with "312 plus 032 plus." In that case, the operator would dial

KP+312+032+ST for the inward that serves 312-456.

It is good practice to ask for "operator's route" specifically, as

there are also "numbers route" and "directory routes." If you do not

specifically ask for operator's route, rate & route will generally assume

that is what you want anyway.

"Numbers" route refers to overseas calls. Example, you want to know how to

reach a number in Geneva, Switzerland (and you already have the number). You

would call routing and say "Numbers route, please, Geneva, Switzerland."

The operator would respond with: "Mark 41+22. 011+041+ST (plus) 041+22"

The "Mark 41+22" has to do with billing, so disregard it. The 011+041

is access to the overseas gateway (to be discussed in Part III) and the 041+

22+ is the routing for Geneva from the overseas sender.

"Directory" routings are for directory assistance overseas. Example:

you want a DA in Rome, Italy. You would call rate & route and say, "Directory

routing please, for Rome, Italy." They would respond with "011+039+ST (plus)

039+1108 STart." As in the previous example, the 011+039 is access to the

overseas gateway. The 039+1108 is a directory assistant in Rome.

2. Nameplace information. Rate & Route will give you the location of an NPA+

exchange. Example: "Nameplace please, for 215-648." The operator would

respond with "Paoli, Pennsylvania." This isn't especially useful, since you

can get the same information (legally) by dialing 0, but using rate & route is

often much faster and it avoids having to hang up when you are already on a

trunk.

*NOTE on Rate & Route: As a blue boxer, always ask for "IOTC" routings. (e.g.,

"IOTC operator's route", "IOTC numbers route", etc.) This tells them that you

want cordboard-type routings, not TSPS, because a blue boxer is actually just a

cordboard position (that Bell doesn't know about).

OVERSEAS COMPLETION OPERATOR (inbound)- These operators (KP+151+ST) assist in

the completion of calls coming in to the United States from overseas. There

are KP+151+ST operators only in a few NPAs in the country (namely 303). To

use one, you would seize a trunk and dial KP+303+151+ST. Then you would

tell the operator, for example, "This is Bangladesh calling. I need U.S.

number 215-561-0562 please." (in a broken Indian accent). She would

connect you, and the bill would be sent to Bangladesh (where I've been

billing my KP+151+ST calls for two years).

Other internal Bell Operators.

KP+11501+ST ...... universal operator

KP+11511+ST ...... conference op

KP+11521+ST ...... mobile op

KP+11531+ST ...... marine op

KP+11541+ST ...... long distance terminal

KP+11551+ST ...... time & charges op

KP+11561+ST ...... hotel/motel op

KP+11571+ST ...... overseas (outbound) op

These 115X1 operators are identical in routing to the 1X1 operators listed

previously, with one exception. If special routing is required (0XX),

then the trailing 1 is left off.

Examples:

A 312 universal op ... KP+312+11501+ST A Franklin Park (312-456) universal

op (special routing 032 required)................... KP+312+03   2+1150+ST

[The trailing 1 of 11501 is left off].

Purposes of 115X1 operators.

UNIVERSAL- Used for collect/callback calls 2+1 to coin stations.

CONFERENCE- This is a cordboard conference operator who will set up a

conference for a customer on a manual operation basis.

MOBILE- Assists in completion of calls to mobile (IMTS) type telefones

MARINE- Assists in completion of calls to ocean going vessels.

LONG DISTANCE TERMINAL- Now obsolete. Was used for completion of long

distance calls.

TIME & CHARGES- Will give exact costs of calls. Used to time calls and

inform customer of exactly how much it cost.

HOTEL/MOTEL- Handles calls to/from hotels and motels.

OVERSEAS COMPLETION (outbound)- assists in completion of calls to oversead  Ls

points. Only works in some, if any NPAs, because overseas assistance has

been centraized to IOCC (covered in Part iii).

Note that all KP+1X1+ST and KP+115X1+ST operators automatically

assume that you are a TSPS or cordboard operator assisting a customer with a

call. DO NOT DO ANYTHING TO JEOPARDIZE THIS! If you do not know what to do,

don't call these operators! Find out what to do first.

This concludes Part II. There is one final part in which I will explain

overseas dialing, IOCC (International Overseas Completion Centre), RQS

(Rate/Quote System), and some basic scanning.

......................................

Better Homes and Blue Boxing

Part III

Advanced Signalling

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

(It is assumed that the reader has read and understood parts I & II before

proceeding to this part).

In parts I & II, I covered basic theory and domestic singalling and

operators. In this part I will explain overseas direct boxing, the IOCC, the

RQS, and some basic scanning methods.

Overseas Direct Boxing.

Calling outside of the United States and Canada is accomplished by using an

"overseas gateway." There are 7 over- seas gateways in the Bell System,

and each one is designated to serve a certain region of the world. To

initiate an overseas call, one must first access the gateway that the call

is to be sent on. To do this auto- matically, decide which country you are

calling and find its country code. Then, pad it to the left with zeros as

required so it is three digits. [Add 1, 2, or 3 zeros as required].

Examples:

Luxembourg (352) is 352 (stays the

same)

Spain (34)   becomes 034 (1 zero added)

U.S.S.R. (7) becomes 007 (2 zeros added)

Next, seize a trunk and dial KP+011+ CC+ST. Note that CC is the three digit

padded country code that you just determined by the above method. [For

Luxembourg, dial KP+011+352+ST, Spain KP+011+034+ST, and the U.S.S.R. KP+011+

007+ST]. This is done to route you to the appropriate overseas gateway that

handles the country you are dialing. Even though every gateway will allow

you to dial every dialable country, it is good practice to use the gateway

that is designated for the country you are calling.

After dialing KP+011+CC+ST (as CC is defined above) you should be connected

to an overseas gateway. It will acknowledge by sending a wink (which

is audible as a <beep><kerchink> and a dial tone. Once you receive internat-

ional dial tone, you may route your call one of two ways:

a) as an operator-originated call, or

b) as a customer-originated call.

To go as a operator-originated call, key KP+ country code (NOT padded with ze-

ros)+city code+number+ST. You will then be connected, providing the country you

are calling can receive direct-dialed calls. The U.S.S.R. is an example of

a country that cannot.

Example of a boxed int'l call:

To make a call to the Pope (Rome, Italy), first obtain the country code,

which is 39. Pad it with zeros so that it is 039. Seize a trunk and dial

KP+011+039+ST. Wait for sender dial tone and then dial KP+39+6+6982+ST.

39 is the country code, 6 is the city code, and 6982 is the Pope's number in

Rome. To go as an operator-originated call, simply place a zero in front of

the country code when dialing on the gateway. Thus, KP+0+39+6+6982+ST would

be dialed at sender dial tone. Routing your call as operator-originated does

not affect much unless you are dialing an operator in a foreign country

To dial an operator in a foreign country, you must first obtain the

operator routing from rate & route for that country. Dial rate & route and if

you're trying to get an operat   or in Yugoslavia, say nicely, "IOTC

Operator's route, please, for Yugoslavia." [In larger countries it

may be necessary to specify a city]. Rate & route will respond with,

"38 plus 11029". So, dial your over- seas gateway, KP+011+038+ST, wait for

sP  pender dial tone, and key KP+0+38+ 11029+ST. You should then get an

operator in Yugoslavia. Note that you must prefix the country code on the

sender with a 0 because presumably only an operator here can dial an

operator in a foreign country.

When you dial KP+011+CC+ST for an overseas gateway, it is translated to

a 3-digit sender code of the format 18X, depending on which sender is

designated to handle the country you are dialing. The overseas gateways and

their 3-digit codes are listed below.

182 ..... White Plains, NY

183 ..... New York, NY

184 ..... Pittsburg, PA

185 ..... Orlando, FL

186 ..... Oakland, CA

187 ..... Denver, CO

188 ..... New York, NY

Dialing KP+182+ST would get you the sender in White Plains, and KP+183+ST

would get the sender in NYC, etc., but the KP+011+CC+ST is highly suggested

(as previously mentioned). To find out what sender you were routed to after

dialing KP+011+CC+ST, dial (at int'l dial tone): KP+0000000+ST.

If you have difficulty in reaching a sender, call rate and route and ask

for a numbers route for the country you're dialing. Sometimes, KP+011+

added country code+ST will not work. I have found this in many 3-digit

country codes. Lexembourg, country code 352, for example, should be

KP+011+352+ST theoretically. But it is not. In this case, dial KP+011+

003+ST for the overseas gateway. If you have trouble, try dialing KP+00+

first digit of country code+ST, or call rate The IOCC.

Sometimes when you call rate and route and ask for an "IOTC numbers

route" or "IOTC operators route" for a foreign country, you will get

something like "160+700" (as in the case of the Soviet Union). This means

that the country is not dialable directly and must be handled through

the International Overseas Completion Centre (IOCC). For an IOCC routing,

pad the country code to the RIGHT with zeros until it is 3 digits. Then KP+160

is dialed, plus the padded country code, plus ST.

Examples:

The U.S.S.R. (7) ...... KP+160+700+ST

Japan (81) ............ KP+160+810+ST

Uraguay (598) ......... KP+160+598+ST

You will then be routed to the IOCC in Pittsburg, PA, who will ask for

country, city, and number being dialed. Many times they will ask for a ringback

[thanks to Telenet Bob] so have a loop ready. They will then place the call

and call you back (or sometimes put you through directly). SoE  me calls, such

as to Moscow, take several hours.

The Rate Quote System (RQS).

The RQS is the operator's rate/quote system. It is a computer used by TSPS

(0+) operators to get rate and route information without having to dial the

rate and route operator. In Part II, I discussed getting an inward routing

for dialing-assistance and emergency interrupts from the rate and route

operators (KP+800+141+1212+ST). The same information is available from

RQS. Say you want the inward routing for 305-994. You would sieze a trunk

and dial KP+009+ST (to access the RQS). Sometimes, if you seize a trunk in an

NPA not equipped with RQS, you need to dial an NPA that is equipped with RQS

first, such as 303. Anyway, after you dial KP+009+ST or KP+303+009+ST, you

will receive a wink (<beep><kerchink>) and then RQS dial tone. At RQS dial

tone, for an inward routing for 305-994 you would dial KP+06+305+994+ST. That

is, KP+06+NPA+exchange+ST. RQS will respond with "305 plus 033 plus". This

means you would dial KP+305+033+121+ST for an inward that services 305-994.

If no special routing were required, RQS would have responded with "305

plus" and you would simply dial: KP+305+121+ST for an inward.

Another RQS feature is the echo feature. You can use it to test your

blue box. Dial RQS (KP+009+ST) and then key KP+07+1234567890+ST. RQS will

respond with voice identification of the digits it recognized, between the

KP+07 and ST.

RQS can also be used for rates and directory routings, but those are

seldom needed, so they have been omitted here.

Simple Scanning.

If you're interested in scanning, try dialing on are a trunk, routings in the

format of KP+11XX1+ST. Begin with 11001 and scan to 11991. There are lots of

interesting things to be found there, as Doctor Who (413 area) can tell you.

Those 11XX1 routings can also be prefixed with an NPA, so if you want

to scan area code 212, dial KP+212+ 11XX1+ST.

There, now you know as much about blue boxing as most phreaks. If you

read and understand the material, and put aside preconceived ideas of what

blue boxing is that you may have aquired from inexperienced people or

other bulletin boards, you should be well on you way to an enlightening

career in blue boxing. If you follow    the guidelines in Part i to box, you

should have no problem with the fone company. Comments made by "phreaks" on

bulletin boards that proclaim "tracing" of blue boxers are nonsense and should

be ignored (except for a passing chuckle).

NOTE 1: CCIS and the downfall of blue boxing.

CCIS stands for Common Channel Interoffice Signalling. It is a signalling

method used between electronic switching systems that eminiates the

use of 2600Hz and 3700Hz supervisory signals, and MF pulsing. This is why

many places cannot be boxed off of; they employ CCIS, or out-of-band

signalling, which will not respond to any tones that you generate on the

line. Eventually, all existing toll equipment will be upgraded or replaced

with CCIS or T-carrier. In this case, we'll all be boxing with microwave

dishes. Until then (about 1995 by current BOC/AT&T estimates), have fun!

If you have ANY questions about this text, please feel free to drop me a

line. I will respond to all mail, messages, etc. Insults are also

welcomed. And if you discover anything interesting scanning, be sure to let

me know.

Mark Tabas

$LOD$

This text was prepared in full by Mark

Tabas for:

K.A.O.S.

Philadelphia, PA.

[215-465-3593].

Any sysop may freely download this text and use it on his/her BBS, provided

that none of it be altered in any way.

Technical acknowledgements:

Karl Marx, X-Man, High-Rise Joe, Telenet Bob, Lex Luthor, TUC, John Doe,

Doctor Who (413 area), The Tone Sweep, Mr. Silicon, K00L KAT, The Glump.

References:

1. Notes on the BOC Intra-LATA Networks

Bell System publication, 1983.

2. Notes on the Network

Bell System publication, 1983.

3. Engineering and Operations in the

Bell System

Bell System publication, 1983.

4. Notes on Distance Dialing

Bell System publication, 1968.

5. Early Medieval Architecture.

.......................................

(c) February 6, 1900         Mark Tabas

.......................................

Call 1-305-994-9966

-- End of File

HOW TO 'BLUE BOX' INTO 'RUSSIA'

BY: MARK TABAS

AS POST   .......ED ON THE GREEK INN BBS

312-774-2035

------------------------------------------------------------------------------

HELLO   ,

LATELY, I HAVE BEEN QUESTIONED ABOUT THE SUBJECT OF BOXING INTO THE SOVIET

UNION.  WHILE I AM BY NO MEANS AN EXPERT IN THIL S, I HAVE PICKED UP A FEW

THINGS.

STANDARD DIALING TO RUSSIA WOULD BEGIN SOMETING LIKE THIS:

KP+011+007+ST   (FOR INT'L, SENDER)

KP+0+95+295+9051+ST   (FOR NUMBER)

IF ALL WENT WELL, YOU WOULD BE ROUTED INTO RUSSIA AS AN OPERATOR-ASSISTED

CALL.  (BY THE WAY, THE NUMBER ABOVE IS THE KREMLIN).  HOWEVER, SINCE ALL IS

NOT WELL BETWEEN US AND THE SOVIETS, YOU WILL PROBABL Y GET A RECORDING.  THIS

WILL GO SOMETHING LIKE, "INTERNATIONAL DIRECT-DIAL SERVICE IS NO LONGER

AVAILABLE TO THE SOVIET UNION-..." OR, "DUE TO CIRCUT CONGESTION IN THE COUNTRY

YOU DIALED, YOUR CALL DID NOT COMPLETE..."

THESE RECORDINGS, HOWEVER, LIE.    THEY ARE MERELY PLACED THERE TO DETER THE

BLUE BOXER.  HE HEARS THEM AND THEN, BELIEVING THEM, GIVES UP.

SEE, THE LAST THI  NG THAT AT&T AND THE U.S. GOVERNMENT IN GENERAL NEED IS A

BUNCH OF 11-YEAR-OLD BLUE BOXERS CALLING  RUSSIA  AND  SAYING  SOMETHBING LIKE:

"HA  HA,  YOU  FORNICATING  COMMY PINKOS,WE'RE GONNA BOMB THE HELL OUT OF YOU!"

SO, THEY MAKE IT *VERY*  DIFFICULT TO BOX THE  CALL.  AS I SAID  BEFORE, THE

RECORDINGS THAT YOU GET ARE FAKES. WHEN NORMAL TSPS PLACE A CALL TO RUSSIA

(LEGALL  Y), THEY MUST CONTEND WITH THE SAME RECORDING, ONLY INTERNATIONAL HAS

THIS NEAT EQUIPMENT THAT KEEPS DIALING THE NUMBER OVER AN D OVER AND OVER UNTIL

THE CALL GOES THROUGH.  THIS NORMALLY TAKES A FEW HOURS.  IT WOULD TAKE YOU A

YEAR ON A BLUE BOX. THIS 4 IS WHY WHEN YOU ATTEMPT TO PLACE A LEGAL CALL TO THE

U.S.S.R., IOCC (TSPS) WILL TELL YOU THAT THERE IS A 2-3 HOUR DELAY.  OFTEN IT

IS MORE THAN THAT, IF THEY HAVE A LINE OF CALLS.

TO HAVE IOCC PLACE A CALL, ONE WOULD BOX KP+160+700+ST FOR RUSSIA.  THIS WILL

GIVE YOU AN IOCC OPERATOR WHO ASKS FOR COUNTRY, CITY AND NUMBER, AND A BILLING

NUMBER (THINKING THAT YOU ARE TSPS YOURSV ELF).  THEN  THEY WANT A CUSTOMER

RINGBACK NUMBER (LOOP OR PAYFONE, IF YOU PLAN TO HANG AROUND FOR A FEW HOURS OR

SO).  HOWEVER, ON CALLS TO OTHER COUNTRIES, THEY WILL CONNECT YOU DIRECTLY AND

DO NOT REQUIRE RINGBACK.

IOCC COUNTRY ROUTINGS ARE AS FOLLg OWS:

KP+011+XXX+ST, WHERE XXX IS THE COUNTRY CODE PADDED TO THE *RIGHT* WITH  ZEROS.

TO CONTINUE ABOUT IOCC ROUTINGS, THEOUNTRY CODE IS PADDED TO THE RIGHT WITH

ZEROS.  THUS, AUSTRALIA (COUNTRY CODE OF 61) WOULD BECOME 610, AND THE ROUTING

WOULD BEa  KP+160+610+ST.

THIS MESSAGE HASN'T SAID MUCH SINCE I HAVEN'T TOLD HOW TO BOX A CALL TO

RUSSIA, I'VE JUST TALKED ABOUT ALL THE PROBLEMS WITH CALLING THERE.  THAT'S

BECAUSE IT IS EXTREMELY DIFFICULT TO DO.  UNLESS YOU ARE VERY PATIENT AND FEEL

LIKE TRYING TO BOX IT ABOUT 5000 TIMES BEFORE YOU FINALLY GET THROUGH (IN WHICH

CASE IT WOULD BE BUSY ANYWAY) THEN YOU NEED THE HELP O F THE BELL SYSTEM OR

POSSIBLY SOME INFERIORS LOCATED THROUGHOUT THE IRON CURTAIN.

TO PLACE A CALL TO RUSSIA, OTHER THAN GOINK G THROUGH IOCC, ONE MIGHT TRY TO

CONTACT SOME SORT OF INWARD SOMEWHERE, LIKE YUGOSLAVIA (KARL MARX'S

SUGGESTION), OR MAYBE EASL T GERMANY.  TO BEGIN WITH, CALL RATE & ROUTE FOR THE

IOTC ROUTING (SAY YOU ARE ON CORDBOARD) FOR INWARD IN YUGOSLAVIA.  SHE'LL  SAY

SOMETHING LIKE "038 PLUS BLAH-BLAH-BLAH" SO YOU WOULD BOX KP+011+038+ST, WAIT

FOR INT'L TONE, THEN DIAL KP+0+38+BLAH BLAH   BLAH+121+ST.  THAT SHOULD GET YOU

INWARD, YUGOSLAVIA, WHO SPEAKS LITTLE OR NO ENGLISH AND IS AT BEST UNPLEASANT

TO DEAL WITH.

THAT'S ALL I KNOW.  IF YOU HAVE A BETTER WAY (OR ANY WAY AT ALL) TO CALL

MOTHER RUSSIA, LET ME KNOW.