|

|        //

|      \X/ Flowerpower *proudly* presents

|

-----+------------------------------------------------------------------------

|

|    a 14 Bit low level sound driver for CyberSound audio sub system

|

EMAIL: flowerp@eikon.e-technik.tu-muenchen.de

INTRODUCTION

------------

This is a driver to replay a 16 bit Stereo audio stream via the Amiga audio

channels  in  14  bit  quality.   It  will  adapt to different screenmodes,

allowing  up  to  60khz output frequency on AGA machines and about 28khz on

ECS  machines.   For  maximum  sound  quality, the driver can be calibrated

using  a  prefs  program.   The  core  routines are written in assembly for

maximum speed, but optimizations are always possible.

HARDWARE/SOFTWARE REQUIREMENTS

------------------------------

An  Amiga  with  at  least  Kickstart  2.0  and  a MC68020 processor should

suffice.   The  driver  takes  approximately  150kb  of  memory, mainly for

tables.   Chip  memory  usage is neglectable as this driver uses very small

buffers and is heavily interrupt driven.

CALIBRATION

-----------

Calibration  of  the  driver  will decrease the noise level and improve the

sound  quality.  Every Amiga has got a different characteristics in its D/A

converters.   They  are not exactly linear.  The calibration program allows

you  to  measure  out  the  step sizes of the D/A converter.  So the 14 bit

driver can create an optimal conversion table.

Calibration  will take approximately 20 minutes.  If you want to do it very

exactly you may invest 2 hours as well ;-)

The  program  will  explain  itself.  Please follow the instructions of the

program in order not to damage your HIFI equipment.

THEORY OF OPERATION

-------------------

14  bit  output  is  created  by  mixing a channel at maximum volume with a

channel  playing at minimum volume.  As this is done on the left and on the

right side, it takes all 4 audio channels.

PROGRAMMER'S DOCUMENTATION

--------------------------

This  is  not  yet  available,  as  this  driver  is not yet finished.  The

software interface may change at any time.  If you are interested in how it

works,  look  into  the source code of the CDPlayer program.

In  the  future the cybersound.library will be responsible for creating the

audio stream, so direct programming of this driver will be useless.

Currently  the  driver is limited to LITTLE ENDIAN audio streams as created

by  the Toshiba CDRoms.  As this is not the best stream format for realtime

mixing on the Amiga, a stream format selection will probably be included.

The source code is included for "educational" purposes only.

FUTURE ENHANCEMENTS

-------------------

* miscellaneous changes in the software interface

* further optimizations for special stream formats

* program a Toccata and Maestro driver as well

PROGRAM HISTORY

---------------

Sounddriver 1.1

* The  audio  hardware was reset even when the driver failed to open.  This

could disturb other programs, e.g. module players. Fixed.

* The  audio  stream  format is now selectable.  The application can switch

between  big  and little endian.  The CDPlayer uses little endian because

the  CDRom  delivers that data format.  A library like cybersound.library

would of course use big endian (motorola format).

Calibration 1.1

* Fixed some enforcer hits that occured when saving the configuration.  You

really  shouldn't  peek into a  menu  structure  that  has  already  been

deallocated.  ;-)

CONTACT ADDRESS

---------------

EMAIL: flowerp@eikon.e-technik.tu-muenchen.de

FidoNet: 2:2480/465

SnailMail:

Christian Buchner

Ganghoferstra�e 2

82110 Germering

Germany

Tel.: +49-(0)89/844872