*******************************************************************************

**********************    FlaySID (Falcon Plays SID)    ***********************

*******************************************************************************

*           A C64 soundchip emulator for Atari Falcon 030 computers.          *

*              Version 2.0. Released on the:  22th of December, 1997.         *

*******************************************************************************

*             (c) 1995-1997 by David Carr�re and Mikha�l Myara.               *

*******************************************************************************

-------------------------------------------------------------------------------

SUMMARY / SOMMAIRE

-------------------------------------------------------------------------------

Please, look at the end of the english section, the chapter called "What's new

in this version ?" / Une version fran�aise de cette documentation est pr�sente

dans la seconde partie de ce fichier, regardez le paragraphe intitul� "Quoi

de neuf dans cette version ?".

A- ENGLISH VERSION / VERSION ANGLAISE

I-   What is FlaySID ?

II-  Some information about system compatibility

III- How to use FlaySID ?

IV-  Known bugs and technical information

V-   Possible questions you may have...

VI-  How to contact authors ?

VII- What's new in this version ?

VIII-For future versions...

B- COURTE VERSION FRANCAISE / SHORT FRENCH VERSION

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

A -  ENGLISH SECTION / VERSION ANGLAISE

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

I - What is FlaySID ?

---------------------

FlaySID is a musicplayer, but not a simple musicplayer : it is able

to play musics from the famous Commodore 64, a little but powerful computer

developped in the beggining of the 1980's.

To play such music-files, FlaySID has a C64 emulator that executes original

C64 music routines (ripped from games, or demos) in real time. The DSP is

widely used to emulate the SID (C64's Soundchip processor).

More than 6000 SID files are available ! If you know somebody who uses PlaySID

on Commodore Amiga computers, or SIDPlay on PC Computers, you can replay their

datafiles : FlaySID is able to load them.

II - Some informations about system compatibility

-------------------------------------------------

FlaySID is a GEM application, Magic, NVDI and MinT compatible (Warning: To

be MinT compatible, you must remove the memory protection, changing the name

MINT.PRG into MINTNP.PRG).

It can be launched as a GEM application (.PRG) or as a desk accessory (.ACC).

FlaySID shares system-ressources with others applications but slows down the

computer, because of the C64 emulation.

FlaySID works on Falcon 030 and should work on compatible computers (with a

DSP56001).

FlaySID works fully with high-speed CPU card (Contact us if you have problems)

even if you change CPU speed when playing.

FlaySID takes about 450Ko of ST RAM when loaded.

FlaySID works with any graphic resolution, but it looks better with 16

colors standard GEM palette, and 80 col. graphic modes (>640 pixels wide).

You should not use FlaySID in Truecolor modes, because they are too slow...

III - How to use FlaySID ?

--------------------------

-----------------

|To load FlaySID|

-----------------

You must have the executable file (FLAYSID.ACC OR FLAYSID.PRG) and the

ressource file (FLAYGEM.RSC) placed in the same directory. To use FlaySID

as a desk accessory, just rename FLAYSID.PRG as FLAYSID.ACC and copy

it (with FLAYGEM.RSC) at the root of your bootdisk (C:\ most of the time).

-----------------

|The main window|

-----------------

This is the first window you have when FlaySID is loaded (or when you call

FlaySID using the Desk Accessory menu).

FlaySID works like many CD players, so you have buttons to :

- Load a file (EJECT button)

- Stop playing a music (STOP button)

- Start playing a music (PLAY button > )

- Pause the music when playing (PAUSE button  || )

- Forward the music faster (FAST FORWARD button >> )

Click on the FlaySID logo to have informations about authors and program.

Click on the "Opt." Button to change some play-parameters

Click on the "Osc." button to display SID's waveforms and registers.

Click on the "+" or "-" button to change the number of the tune you want

to play (a SID file often includes several tunes).

A status line is in the lower border of the main window, it gives you

information about the selected operation or about internal errors.

The "Name", "Author" and "Copyright" fields give information about the

Author, the name of the music, and the copyright (if available in the

music file, else you will have a "<?>").

You can close the window by clicking on the upper-left button (cross).

If you use FlaySID as a desk accessory, the window disapear but FlaySID stays

in memory, and if necessary, continue to play a tune. If you use FlaySID as

a normal executable program (.PRG file), you will quit it.

You can also iconify FlaySID using the upper-right button of the window

("fuller" button). You have then a smaller interface with only the basic

functions. To display again the full window(s), click on the same button.

In 40 col. graphics modes (<=320 pixels wide) FlaySID will automatically

set the main window to its smallest size, and you can not change this

(because there is not enough room space to display the entire interface).

These two buttons exists on all windows (The upper-left button quits FlaySID

only with the main window, otherwise it simply close the window). Closing the

main window will automatically close all the others windows of FlaySID that

you may have opened.

-------------------

|The Option window|

-------------------

This window is very important and contains a lot of parameters for the SID

Player, and for the general use of FlaySID. In this version, only some

options are working.

In the upper part of the window you have buttons called "Emulation",

"Osc.", and "Sound". Clicking on a button will display the options

corresponding, in the middle part of the window.

In the lower part of the window you have three buttons:

"Save" to save the complete actual configuration,

"Cancel" to restore the old configuration of the current options,

"Set" to use the new options choosen.

But, in this version, due to a lack of time, the Save and Cancel options

are not working. In fact, you do not need to click on the SET button to use most

of the options: their effect is instantaneous. But for emulation modes, you must

press the "SET" button. It is not possible to switch from one to another emulator

in realtime, because each of them requires different initialisations and uses

different parts of code, so the new mode will be used the next time you play a

music (pressing start button or changing music).

The Emulation sub-window:

-------------------------

This allows you to change some emulation options. There are two parts:

6502 emulator options:

*"C64 Type" Allows you to change the frequency of the emulated C64. PAL for

european C64 (0.9 MHz), NTSC for US C64 (1 MHz). This will speed up the

the SID (but not the tempo of the music... maybe we should do it also ?!).

The PAL/NTSC option must be used carefully: Use always the PAL option to

play correctly most of the tunes (FlaySID then simulate an european C64).

But if you know that a music was specially done for american C64, then (in

this case ONLY) change for NTSC.

*"Emulation modes" Allows you to change the emulator used. "Fast PlaySID" is

the default mode. This is a new, optimized 6502 emulator. (For programmers:

memory overflow is not managed if the 6502's PC go below $0000 or after $FFFF,

but the playroutine will be stopped - Theorically, all PlaySID ripped musics

will perfectly work). "Normal PlaySID" is the old 6502 emulator of FlaySID

v1.2. We let it here, just to prevent some bugs of the first mode. It will

disappear soon. "SIDPlay PC" uses an enhanced 6502 emulation, wich is fully

compatible with the famous PC SID Player (Bank switching emulation), but

slower than the default mode. Use always the Fast PlaySID mode first, and only

if it does not works, try this mode. "Total C64 mode" let you... well, does

it really worth the price that we explain the aim of this option ?! :)

Of course, a 40Mhz Falcon is highly recommended...

In FlaySID v2.0 the "SIDPlay PC" and "TOTAL C64 mode" are NOT yet working.

*"When no enough CPU time to play correctly": If your Falcon is not fast enough,

to play a music, then FlaySID has three choices: Stop playing ("Error" button),

Just warns you that the sound is not accurate ("Warning" button), or do nothing

and continue ("Nothing" option).

In FlaySID v2.0 the default option is "Nothing" and you can not change this.

*"Consider emulator crashed if execution time is greater than xx seconds".

A playroutine usually lasts 0.02 seconds (50Hz interrupt). But, if something

goes wrong, the 6502 emulator will never ends. And as FlaySID is based on an

interrupt, it will never end, and so... your Falcon will be locked too !

Exactly like a real C64... What a wonderful and accurate emulation ! :-)

But FlaySID is able to see that there is a problem and stops the interrupt.

The problem is for some tunes (ex: those ripped by "Helmond Cracking Service")

whose first routine called ("Init routine") lasts for some seconds (What did

the program do ? Maybe unpacking...). So, you can parameters the maximum time

a 6502 routine can last. The default time is 5 seconds.

In FlaySID 2.0 you can not change this parameter.

DSP SID emulator options:

*"Replay frequency": You can choose among several replay frequencies. The

default frequency is 25Khz and plays 99% of tunes at correct speed on a

standard 16Mhz Falcon. 33Khz frequency is for lucky owners of 40Mhz DSP.

20Khz frequency is really awful, but it is better than nothing if the

DSP has not enough time to play a tune...

*"Emulation of test bit": The test bit is often used to reset or stop

immediately waveforms. A lot of tunes use it. The test bit emulation

of FlaySID is not 100% correct, and this could lead to noisy clicks

sometimes (in fact, this is not really the test bit emulation wich is bad,

but it is the final SID output !).

In v2.0, the default option is "Yes" and you can not change it.

*"Emulation of filters": allows you to choose to emulate SID filters or

no. In FlaySID v2.0 there are not yet filters, so you have to wait for

these options. My God ! How could I live without filters ?! ...Don't

worry, it really worth the price waiting...

The Osc. sub window:

--------------------

This window is only for the time-space viewer.

*"Display SID waveforms": Allows you to see the waveforms of the SID on each

voices, plus the sample voice used sometimes (this sample voices was created

changing quickly the general volume level).

*"Display SID Registers": Allows you to see the main registers of the SID

(so you can understand why some tunes are so wonderful ?!). On the

time-space viewer, below waveforms, you have the names of internal registers

followed with a button. A coloured button means that the register is selected.

The time-space viewer is refreshed when the system has the time to do it.

With tunes wich use a lot of CPU time, the refresh rate may become absolutely

unbearable ! If you have NVDI, this window will be more interesting because

more acurate.

*"Waveforms redraw routines": You can choose among two drawing routines:

-Pure GEM: Only standard GEM routines are used. The main advantage is that

this will probably work on any thinkable configuration, with any graphic

resolution. The main default is that it is so slow that you will forget it

for the next button:

-Gem Compatible: Assembler routines draw waveforms, then a GEM routine copy

them onto the screen. It is fast, and system friendly. It should work with most

of the graphic configurations.

nb: This option is valid only for waveforms, not for SID registers. The display

of the internal registers is always refreshed using standard GEM routines, that is

why they are so slow. Mikha�l know that and will improve that soon.

*"Linked points": You can link or not the points of the waveform display windows,

so that square waveforms will be really square, and not only two parallel lines !

*"Colors for oscillators": Just for the fun !

The "Sound" sub-window:

-----------------------

*"SID Emulation processor"

Here, you can choose to use the Yamaha (soundchip processor of the Atari ST and

Falcon) instead of the DSP to play the music. IN THIS CASE, IT IS NOT REAL SID

EMULATION !! This option is here for the fun. The only way to do real SID emulation

is the use of the DSP Output.

*"Unlock sound-system"

If an application wich use the sound system has a problem (the kind of EXPLOSIVE one !)

it can not end properly, and this could make the sound-system unavailable for

other applications. Clicking on this option will force the sound system, even if it

seems not to be available. Warning, never do it "just to see" if another

application is currently using the sound system in a normal way.

-----------------------

|The Oscillator window|

-----------------------

Opening this window, you will have the digital output of the SID (Three

synthetized voices + the optional digitized voice), and the content of

some SID registers in something we can name "realtime" for an human being

(...but certainly not for a computer !).

You can also display SID registers and/or SID waveforms. To change these

parameters, look at the "Osc." button onto the options window.

IV - Known bugs and technical informations:

-------------------------------------------

We need your opinion to make FlaySID sound (... or look) better. So,

please, tell us all the bugs you could have noticed. This is very important

we can not test the program on every possible configuration !

Some users do that, and so help us a lot.

For SID Emulation/C64 emulation bugs, please, write to David,

and give the name of the musicfile and describe precisely the bug or

the problem. If you are a C64 music coder, and if you think your

playroutine doesn't works correctly, please let me know !

For GEM Interface bugs, write to Mikhael, give your hardware

configuration, the operating system you use (Magic, GEM, ..), the name of

the programs you have in your AUTO folder (same for desk accessories),

and of course, tell your problem !

YOU ARE ALSO ALLOWED TO WRITE US JUST TO TELL THAT IT WORKS FINE ! :-)

*6502 emulator v 1.2:

- Fast PlaySID and Normal PlaySID emulation mode

- Emulation of all legal instructions

- Emulation of some undocumented instructions.

- Cycle-based emulation (Also, cycle-based tempo).

- Emulation of PlaySID's CIA-IRQ/TIMER-A

- Memory managment 100% PlaySID compatible

- Managment of PlaySID extended registers (for Sample & Galway fixed routines).

- In this version, if the emulator find an illegal (undocumented) instruction,

wich is not yet emulated, the playroutine is imediately aborted.

*DSP SID emulator v 2.0:

- REALTIME synthesis at 33, 25 or 20Khz, 16 bits mono, calculated on 24 bits.

- SID emulation accuracy: 30, 40 or 50 6502 cycles (depending on replay

frequency). SID registers are read each 30 (or 40 or 50) cycles. (A

modification of SID registers in an interval lower than 30 (or 40 or 50)

cycles will be take in count).

- Cycle-based emulation of the 4 waveforms and their parameters, with the same

resolution than the SID (However, waveforms are slightly different compared

to the real SID, because FlaySID does not emulate its analogical output).

Good emulation of special waveforms (mixed waveforms), but not very good

(cycle based, waveforms are taken from original C64's samples).

- Cycle based emulation of white noise waveform. It should be close to the

original one, but does not seems to be the same as expected.

- Cycle-based emulation of ADSR registers and volume enveloppes.

- Full emulation of Synchronization and Ring-modulation (FlaySID use the same

algorithms than those in hardware in the SID)

- Good emulation of "Test bit", but not yet perfect.

- Emulation of read-only registers ($d41b, $d41c) but not cycle-based.

- Cycle based emulation of Samples/Galway-noise (PlaySID fixed routines) using

the DSP (accuracy: 30, 40 or 50 C64 cycles, depending on replay frequency).

*YM-2149 Yamaha soundchip processor SID emulator V1.0�:

-Realtime conversion of SID datas into YM datas.

-Only one waveform instead of the three of the SID, plus White noise

-No Ring Modulation

-No Hard Sync but an ugly pseudo-sync effect instead.

-Full emulation of ADSR (using the 68030 to control the YM in realtime)

-Emulation of read only registers

-Emulation of Sample/Galway Noise (PlaySID fixed routines) using the 68030.

-Full emulation of Windows 95 bugs ! :)

*GEM Interface v 1.0� :

- Full XFormDo (Window-based Formulars)

- Contextual Mouse Cursors

- One UserDefined Object has been Defined (200)

- Accessory Messages supported

- Objects Redraw in background windows available.

Comments and known bugs:

-----------------------

*To play at 33Khz you MUST use a hi-speed card. Maybe later this option

will work on a standard falcon...[In fact, sometimes the 33Khz

option works on a standard Falcon, but you have to try it...].

*There are a LOT of new features on this version of FlaySID. It seems

to work fine, but we can not test the program on every thinkable

configuration. If a crash occurs on your Falcon using FlaySID, check first

if there is not a problem of compatibility with another resident program.

If it always crash the OS, then write us ! Warning: sometimes, when FlaySID's

internal 6502 emulator crash, it could take some seconds for the "security

routines" to stop it, so your system seems to be halted for some seconds, but

it is not crashed !

*New FastForward routines speed up the 6502 and SID frequency. But, as the

emulator is splitted between the 68030 and the DSP, if one of the two processors

have not enough time to complete the emulation, of the tempo or the frequency,

only one will be higher creating strange effects ! ...Buy a 40Mhz card !

*Sometimes you could have some noisy bugs in the synthetised sound (a kind of

"click"). This could be normal, and a C64 would also play these clicks, but

it could be that FlaySID imitates the numeric "output" instead of the

analogical output of the SID. We have to lead further analyses on the SID's

analogical output before emulating it.

*Under Magic, no 3D Look Support in Monochrom Modes.

*Iconify Message not yet implemented.

*Some display bugs in the time space viewer using some graphic resolutions

*Sample routines from the YM Player are not stable... they could lock your

computer with some tunes... avoid using them when you do important work

with your Falcon.

*Even if the 6502 emulator has been optimized, FlaySID is not yet able to play

all musics at their real tempo (~ 5% of all available musics) on a basic

Falcon 030 running at 16Mhz. This is because the 68030 has not enough time to

do the emulation ...or because the emulator is not perfect :). In this case,

FlaySID slow down the tempo (/2) to let the 68030 complete the emulation.

If this problem occurs with samples, they will be distorted (DSP Player only).

Happy owners of Hi-Speed CPU card will never have this problem !

If you have only a standard Falcon with 16Mhz 68030 (...like us !) try to

decrease the number of colours of your GEM desktop, so that the graphic

processor ..VIC... hmmm no, VIDEL ;) of the Falcon will steal less memory

read cycles to the 68030 (the best solution is to use a screen saver wich

completely stops the VIDEL, such as TURBO VEILLE). Also, avoid moving the

mouse, because mouse system routines take HUGE time !

*This version is not able to replay tunes from SIDPlay PC wich use the

6510 MMU and/or the CIAs. These features are specific for SIDPlay (and are

probably not compatible with PlaySID Amiga). We are in the process of adding

these features to the emulator.

V-   Possible questions you may have...

---------------------------------------

Q: Why does FlaySID takes so much memory in the new version ? (450 Kbytes)

A [David]: There are obvious answers: the new version includes C64 ROM (20Ko)

(for future MMU emulation) and there is a lot of new code (for the emulator

and for the interface management).

Q: Why the DSP isn't able to do SID emulation at 33Khz on my standard 16Mhz

Falcon ?! All soundtrackers play at 50Khz !

A [David]: I  am a 68030 coder. The DSP Player is the first and the only one DSP

program I ever did. I learn a lot of things about DSP programming each new week,

and it takes time to re-do all the old routines. Some of them are really awful...

I know it, but I can't make about 2500 lines of DSP code in a single week (My

brain is older than this good old 6502 processor !). I must also keep in mind

to improve the 68030 source code (6502 emulators, Yamaha player, and more).

I think the synthesis program will fully work at 33Khz on standard Falcon one

day.

I could do a faster program using general synthesis algorithms. But I don't want

it. It is not interesting for me: I don't want "a" SID synthesis, but THE SID

synthesis (...if possible !). This entails a lot of precalculated tables (illegal

waveforms, ADSR "enveloppes", filters parameters), and a lot of special tests (for

test bit, for special ADSR programming, for SID registers refresh delays). The SID

synthetizer also use some original SID algorithms for waveform generation. And the

problem is that these features use a lot of memory moves and tests, but not a lot of

computing instructions...and the DSP is really not at ease with this kind of

programming... and I am not at ease with the DSP ! :)

To conclude: I choosed MAXIMAL SID FIDELITY instead of MAXIMAL REPLAY QUALITY.

Q: Why is your YM so poor ?! There are plenty of bugs !

A [David]: When I did FlaySID, and when I didn't know how to code the DSP, I

need to test the emulator to see if it worked correctly. I decided to use the

Yamaha to make a fake SID emulation. When the DSP synthesis program was finished,

I decided to let the Yamaha player, because... it was really funny ! So the YM

player is very rough, because it was for my own tests...

The interrupt reproducing the ADSR runs at 200Hz only, even if it is enough most

of the time (the fastest attack rate would need a 8000Hz interrupt to be correctly

reproduced but it takes too much CPU time). The way I do ADSR is not really clever,

I think there is another way to do them. There is no hard sync emulation, but

the hard sync is replaced by... something completely different but wich

could give... well... an idea of what the sound could be... (even if most of

the time it sounds awful :-) ). Concerning the ADSR, I found why it sounds so

poor, I need time to re-do all the ADSR routines ! Argh ! %-)

Comparing the YM player and the DSP player is useful to check if a bug come from

the 68030 emulator (The bug could be heard in the two modes), or from the DSP

(the bug is here only using DSP, but not with the YM).

If users ask for it, I will improve the YM player...

Q: Why Flay uses a cycle-based emulation of the C64 ? Does it really worth the price

and doesn't it slow down the system ?

A [David]: Flay uses a cycle based emulation because it contributes to improve the

fidelity of the SID emulation. The SID create sounds, but a clever "musician-programmer"

can create highly complex sounds changing SID registers at small and/or regular

space of time. Furthermore, if FlaySID adresses the SID using the same timing than

a real C64, I think you will agree that the result will be probably closer to the

reality - (even if you can't often hear the difference...)

So, I think that it worth the price all the more that it doesn't takes extra

CPU time for emulation (except for samples). Having or not the cycle based

emulation will change nearly nothing for the 68030 processor, because it is the

DSP wich do all the required synchronizations. Concerning samples, this is

true that the method used take (not so) much time than an ordinary 68030 interrupt

based routine but I had reasons to choose it:

*I never succeed in playing a sample using a 68030 interrupt and DMA without

having sample distorsion when you move the mouse.

*Using a 68030 interrupt decrease the quality of samples, because you lost

cycle-based emulation for them, and furthermore, all the MFP rates (MFP = Falcon's

interrupt controller) does not cover exactly all the sample rates of the C64.

*With the actual method, FlaySID play the samples exactly like the original C64

and is -theorically- also able to replay ORIGINAL C64 sample routines. I wrote

"theorically" because in fact, as FlaySID does not emulate the C64's CIAs

(=C64's interrupt controllers) it cannot execute original samples routine yet,

but when CIAs emulation will be complete, be sure that it will work !

Q: Do you know the HV-SIDs collection ? I cannot use the v2.0 because it is

in long names fileformat !

A: Yes, of course, we know HV-SIDs. This is the most comprehensive SID tune

collection ever done. Until the v2.0, ZIP files were in MS-DOS fileformat.

But, for the v2.0, authors choosen long name fileformat (Unix, Amiga, Mac

OS), we can not blame them for this choice, it is a normal evolution.

So, there is a problem for TOS Falcon users.

If you use MagX! 5.0 you should not have this problem, because MagX! use

the Windows95's FATname system (If you can test for it, please tell us if

FlaySID works correctly with long filenames). For TOS users would like to

let FlaySID use directly the .ZIP files, so you will not have to unpack them

on your harddisk, and you will have long filename, like everybody !

Jo'ogn Oppermann also make programs to run HV-SID's update files on Falcon.

Contact us if you are interested in.

Q: What ?! You talked about a Total C64 Mode ?

A: Yes.

Q: Does it means that I will see on the screen of my Falcon "C64 Basic...

Ready" with a typical blue background ?

A: This is what I [David] would like to do. But not imediately. We have so much

to do before... But, if you encourage us, maybe things will speed up !

VI- The authors:

----------------

FlaySID is POSTCARDWARE ! You can copy it freely, but you MUST NOT

change the contents of the program/data files/original archive.

...We received 2 postcards until now, and we think we have more than two

users (one in Germany, the other in Poland) in the world. Show us you like

our work, send us a postcard ! Or a photograph of your dog using FlaySID,

or... whatever you want !

Ideas, or questions are welcome !!

*To discuss about 6502 emulation, SID emulation, DSP and 68030 programming,

system compatibility or about the C64, please contact David:

email: David.CARRERE@ima.uco.fr

Ordinary mail:     David CARRERE,

(until June 1998)  cz Mr et Mme Cormier,

26,Rue de Roc Epine,

49100 Angers.

(From July 1998)   Mas du Cres,

30140 BOISSET ET GAUJAC,

FRANCE.

Minitel (For french people only): I threw my Minitel into the trash !

IRC: it is impossible for me to go on IRC. Sorry.

*To discuss about GEM interface programming, C programming and about numeric

filters, please contact Mikha�l:

email: no email ! Maybe one day...

Ordinary mail:     Mikhael MYARA,

13, Impasse du Caroux

34500  BEZIERS,

FRANCE.

Rarely on IRC : "Myself" on #Atari or #Atarifr.

VII -What's new in this version ?

---------------------------------

**Version 2.0, December 1997. fifth update.

Who said we are late ?!! ;-)

-New ! Oscillator window wich displays SID waveforms on each voice

(3 synthetic voices + 1 digital voice) and SID registers.

You can choose among two display routines (system / assembler), you can

view SID registers, choose color, ...

-New ! Option window to change player's parameters.

-Another awful bug removed from the emulator. This was again a bug in the

C64 memory initialization routines (some bytes of the SID file were not

loaded in C64 memory). Universe (by Drax), Chordian (by JCH), and a lot of

tunes like this improvement !!

-Bug removed from the emulation: bad flag refresh after "PLA" instruction.

ex: "Knight games" (Martin Galway) works now correctly.

-Completely new sample routines, now with 100% correct cycle count, and they

should be 100% bug free (...we hope !). The following bugs have also been

eliminated:

-Bug removed from sample routines: with some parameters, the routines wich

compute the samples and send them to the DSP could enter in a never ending

loop ! This locked the Falcon... (ex.: 2400_AD)

-Another bug removed from sample routines (DSP Player). Sometimes, they ended

the playroutine too early. Next playroutines will then completely crash, causing

wrong sound (ex: North&South) or... even worse (Falcon crash !).

-Bug removed from file loading routines. Sometimes, with musics splitted in

two file (*.* and *.INF, or *.DAT and *.SID) the info file wasn't loaded

correctly, so that the emulator doesn't found essential information to

run C64 routines.

-The "tune number" buttons give now all the possible music numbers (0..255).

In older versions, the number was limited to 99, wich was sometimes a problem

(ex: Delta loader). [nb: there is again a bug for high numbers, sorry !]

-According to the resolution of your screen, the icons of the interface will

be converted so that it will look almost the same in any graphic mode.

Welcome VGA users !  :) Note that the interface is a mess in VGA mode

because the system do a bad conversion of objects position... We will

try to improve things...

-New interface features ! You can now iconify the main window of FlaySID to

have just basic commands (load, play, pause, fast forward, Number of tune).

Other very cool features are also available. Enjoy them !

-OPTIMIZED emulator. Now, most of replay routines work at correct speed on a

standard Falcon (16Mhz 68030)... if you don't move the mouse ! If you stop

VIDEL (Video processor) 99% of playroutines work at correct speed, even 500Hz

playroutines.

A lot of things changed in the emulator, we hope it will not be a new source

of bugs for you ! There are two emulation modes in this version: FAST PlaySID

and Normal PlaySID. The latest is here just to prevent some (very very hypotetic)

bugs of the former and will probably disappear soon.

-New management of the tempo of the routines. Now the 68030 depend on to the

DSP (before, it was the opposite !), so the tempo is now really accurate (cycle

based).

-New Fast Forward routines. Don't you think it sounds like a real one now ?

-Huge bug removed from the DSP player: Bad cycle count and bad "synchronisation"

with the 68030 wich could sometimes entails a loss of SID datas, resulting in lots

of sonorous bugs such as what I [David] though to be an ADSR bug. Enjoy now all the

marvellous tunes of Tim Follin and hundreds of other tunes !

**Version 1.2, 14 of April, 1997. Fourth update.

Sorry for the release delay of this update.

-Smaller interface (Useful if you use FlaySID with multitask OS !)

-Bug removed from the player: register $d41c of the SID was always filled with

$00 in DSP mode ! Cause: Silly bug from the DSP player.

-After playing, FlaySID now restores the sound system in a STe compatible mode,

so that STe applications or Falcon applications wich use the Falcon like a STe

will work [Some SID Players will like this improvment... :) ].

-New DSP sound mixing-routines, the sound is now PCM coded, wich improves the

quality (especially white noise) and prevent some little noises when very

short attack/decay times are used.

-FlaySID is now able to reproduce the mixed waveforms of the SID with a good

fidelity: when choosing several waveforms simultaneously on a voice, the SID

mix them in a very special way. So special that nobody knows the real algorithm

to reproduce this output ! That is why, FlaySID use samples of theses waveforms

(We hope to find the algorithm one day...). The output of the following

combinations is exact: Square+Triangle, Square+Sawtooth, Sawtooth+Triangle.

For the Sawtooth+Triangle+Square output, the fidelity is probably lower, because

this output seems to be more complex than expected (Perhaps it vary for each new

waveform ?!). When mixing with the white noise, most of the time the result is

zero because it locks the SID. But in some cases, it could produce some very

short sounds before the SID locks. In this version, the white noise is mixed

with the 3 others waveforms using a logical AND, we will try to do a better

emulation of this combination in a future version !

Examples of mixed waveforms: Storm Warrior or Kanguru.

-33Khz replay option ! This is for lucky owners of CPU and DSP accelerators

(68030 running at 33Mhz/DSP at 50Mhz). I (David) will do my best to make it

works on standard Falcon !

-A lot of improvments and new code into the emulator, to add in a future

version a SIDPlay PC Compatible mode, and a [VERY COOL] mode. Note also

that the actual mode (PlaySID Compatible mode) has been greatly improved.

FlaySID may be now really 100% PlaySID compatible for the memory management !

-Some undocumented instructions added into the emulator.

-Awful bug removed from the emulator. Cause: When FlaySID loaded the SID File

into the C64 memory, the last byte of the SID File was missing. And this byte

was sometimes essential, because it was the $60 instruction (RTS) wich ended

the playroutine !! As a consequence, the emulator executed more code than

expected and furthermore, a random code wich could change the SID Values for

exemple (or completely lock the emulator...), until an undocumented

instruction was met wich ended the playroutine... The results was a lot of

bugs, such as the muting of some voices, emulator crash, etc...

-New security system: FlaySID is now able to detect a crash of the 6510

emulator in PlaySID mode (and in the future SIDPlay mode). Before, this kind

of never-ending routine locked your Falcon (...like on a real C64 !). Now,

if the emulator does not end a routine after 5 seconds, it stops. The "Play"

icon will be selected, but there will not be any sound.

-As you know (?) the InterNet ftp server "ftp.cnam.fr" /pub/Atari/ is dead

:(((. Where could you find new versions of FlaySID ? Try ftp.funet.fi for

exemple, we upload also the new versions here.

**Version 1.1, 6th of January 1997. Third update.

-Improved DSP player: Now, FlaySID plays samples with the same accuracy

than synthesis, i.e. 40 or 50 6502 cycles. You can now MOVE THE MOUSE

without any distorsion of samples !

-Samples are now correctly synchronized with synthetized sound

(This was not the case before !).

-Optimisation of the DSP Player.

-Optimised 6502 emulator: Faaaaaaster emulation !

**Version 1.02�, 16th of December 1996. Second update.

-Improved DSP Player. Now, FlaySID uses EXACTLY the same algorithms than the

SID to make waveforms (except "illegal" waveforms), Hard sync and Ring

modulation.

-Optimisation of the DSP player, you will probably rarely use the

20Khz frequency...

- Bug removed: Quality of sound improved. Cause: Bad synchronization of

DSP and the DMA/CODEC (Bad "DSP mono=>DMA stereo" conversion). The sound

has more high frequencies. Enjoy it !

**Version 1.01�, 2th of December, 1996. First update.

- Bug removed with CIA/IRQ emulation. This bug could lock the computer with

some musics wich use CIA/IRQ (ex: Antiriad). Cause: bad conversion of

CIA/IRQ parameters for the MFP...

VIII- For next versions

-----------------------

Some people though we stopped to improve FlaySID. This is, of course, perfectly

wrong and we have not finished to add hundred of options ! Don't worry, you

will not have to buy a PC to hear at your favourite tunes ! :-)

Future improvements:

*Full emulation of filters

*SIDPlay emulation mode

*Playlist to manage your SID files

*Total C64 mode (what ?!)

*Shell window with useful commands

*A lot of options

*Help window

*...ideas ?

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

VERSION FRANCAISE / FRENCH SECTION

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Cher utilisateur francophone,

Cel� fait plusieurs jours que je dors tr�s moyennement pour terminer avant mon

d�part en vacances cette version 2.0 de FlaySID. C'est pourquoi, je n'ai plus

la force de remettre � jour la version fran�aise de la documentation (pour

cette version uniquement, bien entendu).

C'est pourquoi je vous propose de travailler votre anglais en lisant la

premi�re partie de ce fichier ! Si jamais vous avez un quelconque probl�me

pour comprendre l'utilit� ou le fonctionnement d'une option, alors n'h�sitez

pas � nous contacter pour avoir des pr�cisions.

Je suis bon prince, je vous ai traduit la liste des nouveaut�s sur cette

fabuleuse, que dis-je, extraordinaire (hum...) version 2.0 ! Je vous laisse

aussi les paragraphes d'introduction d'interet g�n�ral.

Qu'est-ce que FlaySID :

-----------------------

FlaySID est un programme rejouant les musiques du c�l�bre Commodore 64, un

ordinateur certes modeste ajourd'hui mais puissant pour l'�poque

(d�but des ann�es 1980).

Pour arriver � ses fins, FlaySID int�gre un �mulateur qui ex�cute les routines

musicales originales du C64 (extraites de jeux ou de demos) en temps r�el. Le

DSP synth�tise parall�lement un son aussi proche que possible du processeur

sonore du C64 (le SID), en fonction des donn�es fournies par l'�mulateur.

Plus de 6000 fichiers musicaux sont disponibles ! Si vous connaissez

quelqu'un qui utilise le programme PlaySID sur Commodore Amiga, ou SIDPlay

sur PC et compatibles, vous pouvez copier ses fichiers musicaux, FlaySID les

interpr�te sans probl�me.

Configuration:

--------------

FlaySID fonctionne sous GEM, et il est compatible avec MagiC, NVDI et MinT

(Mais attention, vous devez retirer la protection m�moire, en renommant

MINT.PRG en MINTNP.PRG). Il peut �tre lanc� comme un programme normal depuis

le bureau (.PRG) ou en tant qu'accessoire (.ACC). FlaySID ne monopolise pas

le temps machine mais le partage avec les autres applications (il tourne donc

en t�che de fond), mais il peut fortement ralentir le syst�me � cause de

l'�mulation du C64.

FlaySID fonctionne sur Falcon 030 ou clones (� condition qu'ils aient un

hardware compatible, notamment au niveau du DSP56001).

FlaySID marche parfaitement avec les cartes acceleratrices, m�me

si vous changez la fr�quence pendant qu'il joue (Contactez nous si vous avez

des probl�mes).

FlaySID prend aux alentours de 450Ko de RAM lorsqu'il est charg�.

FlaySID fonctionne dans n'importe quelle r�solution, mais il est pr�vu pour

fonctionner en 16 couleurs (avec la palette standard du GEM), et avec une

r�solution en largeur sup�rieure ou �gale � 640 pixels. (Utilisateurs de

moniteur VGA, l'interface du programme est d�form�e dans les modes graphiques

avec 400 lignes, il vous faudra attendre une prochaine version avec des routines

d'adaptation automatique [si possible] de l'interface � la r�solution).

L'utilisation de FlaySID en True Color est fortement d�conseill�e, � cause du

temps machine pris par ce mode (l'emulateur pourrait ne plus avoir suffisament

de temps machine pour executer convenablement les routines).

Comment utiliser FlaySID:

-------------------------

Utilisateurs francophones, vous devez d'abord savoir que tous les messages du

programme sont en anglais, ils seront disponibles en fran�ais dans une

prochaine version (�a vous interesse, ou vous y �tes indiff�rents ? Dites le

nous).

Vous devez imp�rativement avoir le fichier ex�cutable (FLAYSID.PRG ou

FLAYSID.ACC selon comment vous l'avez renomm�) ainsi que le fichier ressource

(FLAYGEM.RSC) dans le m�me r�pertoire. Pour installer FlaySID comme

accessoire de bureau, renommez FLAYSID.PRG en FLAYSID.ACC et copiez-le

(...avec son fichier ressource) � la racine de votre disque de demarrage

(C:\ le plus souvent).

FlaySID a un fonctionnement similaire � un lecteur de CD : certains bouttons

pr�sentant les icones conventionnelles sont pr�sents pour :

- Charger un fichier (Bouton EJECT)

- Arreter de jouer une musique (Bouton STOP)

- Jouer une musique (Bouton PLAY > )

- Mettre une musique en pause (Bouton PAUSE || )

- Lire en acceler� une musique (Bouton FASTFORWARD >> )

Cliquez sur le logo "FlaySID" du panneau principal pour avoir des informations

sur les auteurs et le programme.

Cliquez sur le bouton "Osc" pour avoir l'oscilloscope.

Cliquez sur le bouton "Opt" pour d�finir les options du programme.

Cliquez sur les boutons "+" ou "-" pour changer le num�ro de la musique que

vous voulez �couter (En effet, un m�me fichier SID peut contenir plusieurs

musiques).

En bas du panneau principal, vous avez la barre d'�tat qui vous donne en

permanence des informations sur l'op�ration en cours et sur d'eventuelles

erreurs internes au programme.

Les lignes "Name", "Author" et "Copyright" vous renseignent sur l'auteur, le

nom et la date du copyright de la musique (si ces informations sont pr�sentes

dans le fichier, sinon un point d'interrogation "<?>" s'affichera).

Pour le reste des fonctions de l'interface, reportez vous � la version

anglaise.

Bugs connus et d�tails techniques:

----------------------------------

N'oubliez pas que nous avons besoin de votre avis pour faire progresser

FlaySID tant du point de vue de l'�mulation que de l'interface. N'h�sitez

pas � nous dire tous les bugs que vous rencontrez, il nous est impossible

de tester FlaySID dans toutes les configurations possibles !

Pour tous les bugs concernant l'�mulation du C64 ou du SID, contactez David

et donnez le nom du fichier musical ainsi qu'une description compl�te du

probl�me. Si vous �tes un programmeur sur C64 et que vous pensez que votre

routine est mal �mul�e, contactez-moi.

Pour les probl�mes avec l'interface GEM, �crivez � Mikha�l, donnez votre

configuration hardware, le systeme d'exploitation utilis� (Magic, GEM, ...),

le nom des programmes que vous avez dans le dossier Auto ainsi que la liste

des accessoires utilis�s, et bien sur d�crivez le bug rencontr� !

Les auteurs:

-----------

FlaySID est un CartePostaleGiciel (beurk ! �a donne un peu la naus�e de

tout franciser), cel� veut dire que si vous l'appr�ciez, vous nous envoyez

une chouette carte postale de chez vous. Vous pouvez aussi nous envoyer une

photo de votre grand m�re en skate board, ou de votre bidet flambant neuf.

Mais envoyez quelque chose, quoi ! Pour le moment, nous avons re�u la quantit�

�l�phantesque de .... DEUX cartes postales. C'est magnifique.

Vous pouvez diffuser FlaySID librement, mais vous ne DEVEZ PAS modifier le

contenu de l'archive/du programme original/de ses fichiers de donn�es.

*Pour parler de l'emulation 6502, l'emulation SID, de la programmation DSP et

68030, de la compatibilit� syst�me ou au sujet du C64, contactez David :

email: David.CARRERE@ima.uco.fr (valable jusqu'� Juillet 1998 environ)

adresse jusqu'� Juin 1998:

David Carrere,

cz. Fam. Cormier,

26 Rue de Roc Epine,

49100 ANGERS,

FRANCE.

Ensuite :          David CARRERE,

Mas du Cres,

30140 BOISSET ET GAUJAC,

FRANCE.

Minitel (France uniquement) : J'ai jet� mon minitel par la fen�tre depuis

un bon moment.

IRC: L'acc�s � IRC ne m'est pas possible, croyez bien que je le regrette !

*pour parler de la programmation d'interfaces GEM, de la programmation C, ou

des filtres numeriques, et aussi de la compatibilit� systeme, contactez

Mikha�l:

email : ha ben non, 'a pu email !

adresse :          Mikhael MYARA,

13, Impasse du Caroux

34500  BEZIERS,

FRANCE.

Jamais sur IRC : "Myself" on #Atari or #Atarifr.

Quoi de neuf dans cette version ?

---------------------------------

**Version 2.0, Decembre 1997. Cinqui�me mise � jour.

... Qui a dit que nous �tions en retard ?!! ;-)

-Nouvelle interface. Ca se passe de commentaire: regardez, et admirez !

Vous pouvez iconifier la fen�tre principale, ce qui aura pour effet de cacher

toutes les fen�tres de FlaySID. Tr�s pratique quand votre �cran est encombr�.

-Nouveau ! Oscilloscope affichant les formes d'ondes ainsi que les

principaux registres du SID utilis�s, dans ce que l'on pourrait appeler

du "temps r�el" pour un �tre humain. Pauvre Homme.

Vous pouvez choisir entre plusieurs routines d'affichages, et divers

autres zigouigoui bien sympathiques.

-Halala, encore un affreux bug retir� de l'�mulateur ! C'�tait un bug dans

l'initialisation des routines C64. Inutile de dire que les musiques

appr�cient (surtout lorsqu'elles rebouclent au d�but).

-Bug trouv� et atomis� dans l'�mulateur: mauvais rafraichissement des

drapeaux du 6502 apr�s l'intruction "PLA". Ha ben, voil�, maintenant il

marche "Knight Games" de Monsieur Galway.

-Routines de samples compl�tement nouvelles, plus rapides, plus propres, sans

phosphates, et avec un comptage de cycles enfin correct. D'autres bugs ont �t�

mururoaris�s:

-Parfois les routines de sample tournaient en boucle, et le Falcon ne rendait

jamais la main. exemple: 2400_AD.

-D'autre fois, elles arretaient arbitrairement la routine 6502 alors qu'elle

n'avait pas fini. Les playroutines suivantes se plantaient ou bien ne pouvaient

pas fonctionner normallement. Le zozio pouvait m�me se planter. Vous pouvez enfin

apprecier les musiques de "North&South".

-Bug hiroshimas� dans les routines de chargement des fichiers. Pour les musiques

divis�es en deux fichiers (*.* associ�s aux *.INF et *.DAT associ�s aux *.SID)

parfois FlaySID ne trouvait pas le second fichier, et il lui manquait des

informations essentielles pour pouvoir jouer correctement.

-Le num�ro de musique peut d�sormais aller jusqu'� 255, ce que Delta Loader par

monsieur Rob Hubbard appr�cie. Le probl�me est qu'il subsiste un petit bug dans

l'affichage des "grands" nombres (>32). D�sol�. C'est Mikha�l qu'il faut

frapper ! :-)

-Selon la r�solution de votre �cran, FlaySID adaptera ses icones. Bienvenue

utilisateurs VGA... quoi que non, pas encore en fait, car il subsiste une sorte

de "bug" qui transforme une interface soign�e en 200 lignes en un truc un petit

peu immonde en 400 lignes. Zut, il reste du travail � faire...

-Emulateur OPTIMIS� ! Maintenant, la plupart des musiques tournent � la bonne

vitesse sur un Falcon � 16Mhz... si vous ne bougez pas la souris, bien entendu.

Et si vous stoppez le VIDEL (processeur graphique du Falcon) gr�ce � un protecteur

d'�cran bien sympa style Turbo Veille, l� ce sont 99% des musiques qui passent.

Enorm�ment de choses ont �t� modifi�es ou ajout�es dans le programme. Nous esp�rons

que cel� ne sera pas une nouvelle source de bugs pour vous ! Deux emulateurs sont

pr�sents dans cette version de FlaySID: le FastPlaySID, tout neuf, qui va plus

vite, d'o� son nom. Le NormalPlaySID qui va disparaitre vite, et n'est l� que pour

pallier � de (tr�s hypoth�tiques) bugs du premier.

-Nouvelle gestion du tempo des musiques qui devrait enfin �tre strictement le

m�me que sur un C64 (le 50Hz est du vrai 50Hz, et plus du 49.9Hz comme avant, par

exemple). En effet, maintenant le 68030 doit ob�ir au DSP, ce dernier controlant

les cycles d'horloge 6502.

-Nouvelles routines d'avancement rapide de la musique. Ne trouvez-vous pas que

�a sonne comme du vrai accel�r� maintenant ?!

-Gargantuesque bug nagasaki� dans les routines DSP. Il y avait un mauvais comptage

des cycles machine, et une mauvaise synchro entre DSP et 68030, ce qui pouvait

entrainer l' "oubli" de donn�es SID de la part du DSP. Et dire que je croyais

qu'il s'agissait d'un bug dans d'autres routines compl�tement diff�rentes

(ADSR)... Les musiques qui s'en trouvent am�lior�es se comptent par centaines.

Dont les g�niales musiques de Tim et Geoff Follin !

-Vous avez failli avoir le filtre passe bas, j'y ai [David] pass� la nuit derni�re

dessus, mais rien n'y fait, �a buggue, et je ne peux pas le laisser. Dommage.

Holala, Wizball ce qu'il rend avec les filtres ! ...pardon.

**Version 1.2 du 14 Avril 1997. Quatri�mme mise � jour.

D�sol�s pour le temps de mise � jour un peu long...

-Bug retir� du player: Le registre $d41c du SID �tait toujours vide en mode

DSP ! Cause: Bug idiot du player DSP.

- Interface plus petite. Tr�s utile pour les utilisateurs de syst�me

multit�che avec plein de fen�tres partout !

-Apr�s avoir jou� une musique, FlaySID restaure le systeme sonore en mode

compatible STe, de telle fa�on  que les applications STe ou les applications

qui utilisent le Falcon comme un STe fonctionneront [Certains players SID

appr�cieront beaucoup cette am�lioration... :) ]

-Nouvelles routines de mixage du son au DSP, le son est maintenant cod� en PCM

ce qui augmente la qualite du son (surtout le bruit blanc) et �vite quelques

petits bruits (sorte de craquements) lorsque des enveloppes de tr�s courte

dur�e sont utilis�es.

-FlaySID est enfin capable de reproduire les ondes sp�ciales du SID avec une

bonne fidelit�: Lorsque plusieurs formes d'ondes sont choisies sur une m�me

voix, le SID les m�lange de fa�on tout a fait �trange. Tellement bizzare,

que personne  ne connait le v�ritable algorithme pour synthetiser ces

ondes ! C'est pourquoi, FlaySID utilise des samples de ces ondes, faits

directement sur Commodore 64 (Cependant, nous ne d�sesp�rons pas de trouver

l'algorithme un de ces jours). La sortie des combinaisons suivantes est

exactement la m�me que sur le SID: Rectangle+Triangle, Rectangle+Scie,

Triangle+Scie.

Pour la combinaison Rectangle+Triangle+Scie, la fidelit� est probablement

plus faible car ce m�lange semble plus complexe que pr�vu (L'onde semble

varier a chaque p�riode ?!).

Lorsque l'on m�lange le bruit blanc avec une autre onde, la plupart du temps

le r�sultat est nul car cela bloque le SID. Mais dans certains cas, un court

signal utile  peut �tre cr�e avant de planter le SID. Dans cette version le

bruit blanc est m�lang� avec les 3 autres formes d'ondes par un ET logique,

On essayera de mieux �muler cette combinaison dans le futur.

Exemples de musiques utilisant les ondes mix�es: Storm Warrior, Kanguru, etc...

-Option pour rejouer les musiques � 33Khz. R�serv� aux possesseurs de cartes

acceleratrices (68030 a 33Mhz, DSP a 50 Mhz).

-Beaucoup d'am�liorations et de nouveaux morceaux de programme dans

l'�mulateur afin d'ajouter dans une prochaine version un mode compatible

SIDPlay PC, et un mode [SURPRISE]. Notez aussi que le mode actuel

utilis� par FlaySID (Mode Compatible PlaySID) a �t� bien am�lior�.

FlaySID devrait �tre � pr�sent vraiment 100% compatible PlaySID au niveau de

la gestion memoire du C64 !

-Emulation de quelques instructions non-document�es (il en reste encore

un paquet a faire !).

-Affreux bug retir� de l'�mulateur. Origine: Lorsque FlaySID installait la

routine du fichier SID en m�moire C64, le dernier octet n'etait pas

copi�. Or, cet octet etait parfois essentiel car c'�tait l'intruction $60

(RTS) du 6510 qui signifiait la fin de la routine ! L'�mulateur executait

alors plus de code que pr�vu et qui plus est, du code al�atoire qui pouvait

changer le contenu des registres du SID par exemple (ou compl�tement planter

l'�mulateur) jusqu'� ce qu'une instruction non document�e soit rencontr�e

ce qui mettait fin � la playroutine... Cela provoquait donc beaucoup de bugs

tr�s vari�s, comme la coupure de certaines voix, la modification du son,

etc...

-Nouveau syst�me de s�curit�: FlaySID est � pr�sent capable de d�tecter un

plantage de l'�mulateur 6510 en mode PlaySID (et dans le futur mode SIDPlay).

Auparavant, ce type de routines sans fin bloquait votre Falcon (exactement

comme sur un v�ritable C64 ! Mais c'est le genre de raffinements dont on se

passe...). Maintenant, si jamais l'�mulateur n'a pas termin� une routine

apr�s 5 secondes, il est arr�t�. L'icone "Play" sera s�lectionn�e mais aucun

son ne sera produit.

-comme vous ne le savez peut �tre pas, le serveur ftp.cnam.fr /pub/Atari n'est

plus de ce monde :((( "O� vais-je pouvoir trouver les dernieres version de

FlaySID ?!". Pas de panique, essayez ftp.funet.fi o� les nouvelles versions de

FlaySID sont d�pos�es. Sinon, essayez aussi les mirroirs du ftp.cnam.fr qui

sont peut �tre encore valables (ftp.grolier.fr ou ftp.guiva.es / adresses a

verifier).

**Version 1.1, du 6 Janvier 1997. Troisi�mme mise � jour.

-Player DSP am�lior�: A pr�sent, FlaySID rejoue les samples et le Galway

Noise avec la m�me pr�cision que la synth�se sonore, � savoir 40 ou 50

cycles 6502. Vous pouvez d�sormais BOUGER LA SOURIS sans distordre les

samples !

-Les samples sont maintenant correctement synchronis�s avec le son synth�tis�

(...Ce n'�tait pas le cas auparavant !)

-Optimisation du player DSP (Encore ?!)

-Emulateur 6502 optimis�, il �tait temps ! Il reste encore du boulot, mais

�a va d�j� mieux.

**Version 1.02�, du 16 D�cembre 1996. Seconde mise a jour.

-Player DSP am�lior�. A pr�sent FlaySID utilise EXACTEMENT les m�me

algorithmes que le SID pour generer les ondes (except� les formes d'ondes

mix�es), la synchro et la modulation en anneau. Le player DSP a �galement �t�

optimis�, je pense que vous n'aurez quasiment plus jamais � utiliser la

fr�quence de 20Khz, tant mieux, c'est vraiment nul 20Khz ! (d�j� que 25Khz

ce n'est pas le pied...).

-Qualit� du son am�lior�e. Cause: Mauvaise synchronisation entre le DSP et

le DMA/CODEC (Mauvaise conversion des donnees FlaySID/Mono en donn�es

DMA/Stereo). Maintenant, le son est plus riche en aigus (le son de la

precedente version semblait "etouff�").

**Version 1.01�, du 2 Decembre 1996. Premi�re mise � jour.

- Bug avec l'emulation du CIA/IRQ retir�. Ce bug pouvait planter la machine

avec quelques musiques qui utilisaient le CIA/IRQ (ex: Antiriad). Cause:

mauvaise conversion des param�tres CIA/IRQ en param�tres MFP...

------------------------------------- END / FIN ------------------------------