# -------------------------------------------------------------------

# VIDEO                                 (c) Copyright 1995 KKP & Nat!

# -------------------------------------------------------------------

# These are some of the results/guesses that Klaus and I Nat! found  out about

# the Jaguar with a few helpful hints by other people, who'd prefer to remain

# anonymous. HDB2, accurate linebuffer, CLUT info by NEUROMANCER

#

# Since we are not under NDA or anything from Atari we feel free to give this

#  to you for educational purposes only.

#

# Please note, that this is not official documentation from Atari or derived

# work thereof (both of us have never seen the Atari docs) and Atari isn't

# connected with this in any way.

#

# Please use this informationphile as a starting point for your own exploration

# and not as a reference. If you find anything innacurate, missing, needing

# more explanation etc. by all means please write to us:

#    nat@zumdick.ruhr.de  or kp@eegholm.dk

#

# If you could do us a small favor, don't use this information for

# those lame flamewars on r.g.v.a or the mailing list.

# -------------------------------------------------------------------

# $Id: video.html,v 1.22 1997/11/16 18:14:42 nat Exp $

# -------------------------------------------------------------------

T H I S   S H I T   I S   P O T E N T I A L L Y   D A N G E R O U S

T O   Y O U R   T V - S E T   O R   M O N I T O R .

U S E   T H I S   I N F O R M A T I O N   A T   Y O U R   O W N   R I S K.

You better NOT fuck with the video registers, unless you can bear the

disappointment of a broken TV-set or monitor. I'd suggest to read up

on a TV instructors manual beforehand. We disclaim any responsibility

for damage on hardware, because you used this information.

This documentation is at the moment in a transitory state.

Linebuffer	2

Video Modes	2

Controlling the Monitor	2

Calculating the vertical registers:	5

Calculating the horizontal registers:	6

Useful video setups	7

Registers	7

VMODE Video/pixel mode	$F00028 RW	7

BG  Background color	$F00058 WO	8

BORD Border color ($F0002A - WO)	8

HP Horizontal Period / scanline length	$F0002E WO	8

HBB Horizontal Border Begin	$F00030 WO	8

HBE Horizontal Border End	$F00032 WO	9

HS HSYNC position	$F00034 WO	9

HVS VSYNC Horizontal Position	$F00036 WO	9

HDB1 Horizontal Display Begin #1	$F00038 WO	9

HDB2 Horizontal Display Begin #2	$F0003A WO	9

HDE Horizontal Display End	$F0003C WO	9

HEQ Horizontal Equalization Start	$F00054 WO	10

VP Vertical Period / frame length	$F0003E WO	10

VBB Vertical Border Begin	$F00040 WO	10

VBE Vertical Border End	$F00042 WO	10

VS VSYNC position	$F00044 WO	10

VDB Vertical Display Begin	$F00046 WO	10

VDE Vertical Display End	$F00048 WO	11

VEB Vertical Equalization Start	$F0004A WO	11

VEE Vertical Equalization End	$F0004C WO	11

GENLOCK Genlock control	$F00026 W	11

CLUT Color LookUp Table	$F00400 RW	11

HC Current index in linebuffer	$F00004 RW	12

VC Current linebuffer number	$F00006 R	12

VI Video Interrupt control register	$F0004E RW	12

LPH Light pen/Laser Gun Horizontal coordinate	$F00008 R	12

LPV Light pen/Laser Gun Vertical coordinate	$F0000A R	12

LBUFA Linebuffer A	$ F00800 RW	12

LBUFB Linebuffer B	$F01000 RW	12

LBUFC Current linebuffer	$F01000 RW	12

ADDENDUM	13

# Linebuffer

To understand the video chip, you need to understand that the video chip

is not a DMA device. It doesn't have a screen memory it reads the displayable

image from, but only has two small 1440 byte linebuffers, that it uses to

produce the color data for a scanline.

It is dependent on other chips to fill its internal linebuffer with data.

The linebuffer is a 1.4Kb large strip of RAM. It is organized either as

720 16-bit pixels, or as 360 32-bit pixels. The color representation

employed and the size of the pixels can be selected with VMODE.

If a resolution of higher than 720/360 pixels is desired, than you

must use both linebuffers at the same time. (HDB2)

Someone

# Video Modes

With the VMODE register, you can switch between some modes of operation

in the graphics chip.

It is possible to switch between 32 bit pixels and 16 bit pixels.

It is possible to switch between 16 bit RGB modes and 16 bit CrY modes.

There is also a mixed mode, that uses the LSB of a pixel to distinguish

betweeen 5-5-5 RGB color mode and 8-7 CrY mode.

It is possibly possible to switch between interlace and non-interlace.

# Controlling the Monitor

The video chip needs to be set up with all the values you see

marked with three- (or two-)letter codes in the picture, as there are:

HP:      horizontal period

HBB:     begin of horizontal blank

HBE:     end of horizontal blank

HDB(12): start of horizontal display

HDE:     end of horizontal display

HS:      horizontal sync impulse

HEQ:     horizontal EQ impulse (left halfline)

HVS:     vertical sync impulse (horizontal position)

VP:      vertical period

VBB:     begin of vertical blank

VBE:     end of vertical blank

VDB:     begin of vertical display

VDE:     end of vertical display

VEB:     begin of EQ

VEE:     end of EQ

VS:      initiate vertical syncing

As you can see the screen is divided in two halflines, quite like

the Falcon. Please don't ask me what that is good for it is just

the way it is. Each halfline starts at zero and counts up to HP.

The second scanline is marked with a $400. If you understand this

part you'll have little problem following with what is going on.

You have to specify the horizontal frequency in terms of cycles per

scanline (almost) and the total number of scanlines per frame. So

the total time taken for one frame should be:

one_frame = horizontal_cycles * number_of_scanlines

now calculate in the refresh rate and you get

total_cyles = one_frame * refresh_rate

and the other way around

horizontal_cycles = total_cycles / refresh_rate / number_of_scanlines

Now starting from this you design the horizontal and vertical blanking.

The total number of cycles taken for one horizontal blank is:

horizontal_blanking_cycles = start_hblank + end_hblank

The number of potentially visible cycles is

visible_cycles = horizontal_cycles - horizontal_blanking_cycles

The total number of scanlines used in the blanking area is

blanking_scanlines = start_vblank + end_vblank

The number of potentially visible scanlines is

visible_scanlines = number_of_scanlines - blanking_scanlines

In the time the video chip is not blanking it is outputting color

information. The video chip can either output a Border color or

pixel data. From the time the horizontal blanking ends until pixel

output occurs, the video chip will display the Border color. It

will also display the Border color, when pixel output is finished

and the next horizontal blank is not yet initiated.

The video system keeps track of what it is doing with two registers

called VC (for Vertical Counter) and HC (Horizontal Counter). The

following image depicts a 16 scanlines heighted screen.

As you can see each scanline is split into two halflines. Usually

the pixel data is displayed in the left half only!

Lets go through one _visible_ horizontal line step by step.

1. The video chip isn't displaying anything its internal counter (HC)

is set to zero. It is in the blanking phase. It will continue

to be in this phase until ...

2. The HC reaches HBE, the end of the horizontal blank. It now starts

to output the Border color (in BORD) to the DAC until ...

3. HC reaches HDB1, the begin of the display area. The video chip will

now fetch the values from the linebuffer and send them to the

DAC. At the same time the other linebuffer is initialized with the

default color and the OP is called to prepare the other linebuffer

until ...

[  after finishing the first linebuffer HC's bit #10 ($400) will be set,

and the counter be reset to 0.

4. it reaches HDB2 (or possibly not). If it does reach it, then its

basically like step #3. The OP is called also AGAIN!. The OP will have

no idea, that this second buffer is on the same scanline, except if

you program its object list to recognize that fact!

Actually in 99% of the cases HDB2 is just HDB1 or outside of the

scanlines lenfth, so nothing special happens until...

5. HC reaches HDE. At this point the linebuffer is of no interest

anymore and the Border color is displayed again until...

[ Maybe HDE must have bit #10 cleared in double linebuffer (HDB2) mode? ]

[  At this point the OP can now fill the linebuffer for the scanline

two scanlines ahead. It probably is started right at this moment ]

6. HBB, the start of the horizontal blanking period is reached. Now

the DAC is turned off and not much happens, until...

7. HC reaches the HS. Now the horizontal synch signal is sent and

nothing much of interest will happen until we're back at step 1.

We did not use the HEQ and the HVS registers. Well the HVS register

will be used, when we describe the vertical scanline organization in

a bit more detail. My take on the EQ thing is this:

Shortly before the VSYNC, the video chip will send out a few

impulses to synchronize for the next VSYNC and it will do so for a

while after the VSYNC impulses have passed. It seems that with HEQ

you can specify the place where one half of the equalitation impulse

should occur. (The other half occur either at the same position of

the second halfline, or - second theory- at the position of the

HSYNC. I kinda favor the first theory.)

With the VEB and VEE you can prescribe where the EQ should be happening.

For german EE guys, I am trying to match the "Trabantenkonzept"

unto this equalization thing.

Lets go through one frames worth of scanlines step by step:

1. The video chip isn't displaying anything its internal counter (VC)

is set to zero. It is in the blanking phase. It will probably continue

to do equalization (no HSYNCs just EQ pulses) until...

2. The VC reaches VEE, the end of the EQ section. Now it will revert to

sending normal HSYNCs instead of "Trabanten", but it still keeps on

blanking until...

3. HC reaches VBE, the end of the blanking area. The video chip will

now start processing the 'visible scanlines' first only displaying

Border colors until ...

4. it reaches VDB. It will now start processing scanlines as

described above until...

5. VC reaches VDE. At this point we are back to displaying the Border

color. This will continue until VC reaches...

6. VBB, the start of the vertical blanking period. Now the Border

color will not be displayed and not much happens, until...

7. HC reaches the VEB. The begin of the equalization section. Now

the video chip stops sending HSYNCs and starts sending "Trabanten"

or EQ-impulses. This is all of course in preparation for the event

of VC's content matching...

8. the content of the VS register. Now the vertical synch signal

is sent and nothing much of interest will happen until we're

back at step 1.

## Calculating the vertical registers:

The vertical period is the number of scanlines per refresh. Now we gotta

consider a refresh in NTSC terms, there one refresh is split into two

half pictures. The picture is split into even and odd scanlines and

the even lines are shown in the first frame and the odd lines are shown

in the second frame. The VP thinks in NTSC term so even if it is not

running in interlace mode, we gotta prepare it as if it was running in

interlace mode. What does that mean ? Well basically double every value

in sight :)

Except of course the VP register, which is special:

VP:      525      (we already knew that)

Now lets do the VSYNC like this, we need 3 lines for VSYNC impulses

and we double this value so:

VS:      VP - 6 = 519

The equalization should happen three scanlines before the VSYNC and

three lines after, so we set (doubling again):

VEB:     VS - 6 = 513

VEE:     6

Gotta figure, where it all begins. We need to blank at least on the

line of the EQ phase. So lets use this and give two scanlines for

safety.

(Remember the doubling: 2*2=4)

VBB:     VEB - 4 = 509

Now right here we got a tiny little problem because this number is odd,

and odd numbers address in non interlaced mode mean the right halfline

only,so we round down to 508.

VBB:  508

After the VSYNC impulses we need to allow some time for the retrace.

NTSC takes 40 scanlines total for vertical blanking.

VBE:     40 - (VP - VBB) = 23 -> 24

We'll set it up for 200 visible lines and let the Border color the

other lines. For (mathematically) aesthetic reasons, we're going to

center our picture in the Border.

VDB:     VBE + ((485 - 400) / 2) = 66

VDE:     VDB + ((200 - 1) * 2)   = 464

## Calculating the horizontal registers:

You calculate the value for the HP-register like this:

If the Jaguar FAQ is correct, the machine runs with 26.591 Mhz. For an NTSC

TV-set you wanna do 29.97 Hz refreshes a 525 lines, or ca. 60Hz with 262.5

lines. We know that HP covers half a scanline (see the picture) so, we

calculate the length of one halfline in terms of the Jaguars clock:

HP: 26.591 Mhz / 29.97Hz / 525 = 1690 cycles / 2 = 845

One halfline takes 1 / 29.97 / 525 = 63.56 us time.

There's a certain amount of time you have to spend in the horizontal and

vertical blanking period, because the beams retrace speed is something you

can't manipulate. So these numbers are pretty much constant.

H                        H                           H

B                        E  |                        B H

E                        Q  |                        B S

+----v------------------------v--+------------------------v-v----+

/          |         |         |         |         |         |   \

0          10        20        30        40        50        60    63

Lets use:   for the starting blank  4.6 us length

for the ending blank    6.4 us length

lets do the hsync at   59.0 us

and the EQ at          29.5 us (and 61.3 us ?)

This will become a bit mathematical I am afraid:

HBE4,6 μs=HP×263,56 μs⇔HBE=1690×4,6 μs63,56 μs⇒HBE=122

Now HBB will start at position 63.56 - 6.4 = 57.2. Since the video

system is thinking in halflines, we mentally (and mathematically)

transpose the second halfline to the first, and set the bit #10.

So in the first line it would be:	57.2 - 63.56/2 = 25.42

HBB=1690×25,4263,56=676+$400

I'll skip the calculations of most of the other horizontal registers,

because they're analog to the calculations above.

The registers HDB1+2 and HDE, you probably wanna treat a little bit

special because you might like to know how big your playfield gonna

be. Lets assume you want to create a 400 pixel wide playfield.

Lemme check how much time we have left:

scanline - start blank - end blank =

63.56 - 4.6 - 6.4 = 54.56 =

1450 cycles.

Now we have to take into account the VMODE register, which specifies

how many cycles are combined to a linebuffer pixel. Or rather on

how many cycles the linebuffer pixel's value will be output on the DAC.

If we have 1450 cycles left, then divided by 400 yields 3.6. Since we

can't have more than those 1450 cycles we settle for pixelsize 3 and

we'll use up 1200 cycles. The rest of the cycles will be used for the

Border. This may sound wasteful, but you have to consider that on a TV

set you have quite a bit of Border until the beam hits the phosphor.

Now lets calculate HDB. Since we wanna center the picture we'll use

600 cycles on the left and 600 cycles on the right. This isn't perfect

really, you gotta experiment a little...

So HDB1 = HDB2 = HP - 600 and

HDE  = HP + 600 - 1 | $400

Remember that HDB1/2 are on the first halfline and HDE ($400 !) is on the

second.

# Useful video setups

Usually you don't have to fiddle with most of the video registers, since

the Jaguar EPROM on bootup will set the default values for your NTSC or

PAL jag. You probably only want to change the HDB12/HDE and VDB/VDE setting

for overscanning. Note that you shouldn't cross over the HBE/HBB and VBE/VBB

boundaries when doing so. The default values may not be all to your liking

though (ain't to mine), so you might think about it differently.

If you want to run in a different horizontal resolution than 320 or any

reasonable multiple thereof, I suggest you calculate the maximum number of

cycles you have between the blanks, divide by the pixelsize you want and

then choose a number from what you got, with which you feel comfortable.

Finally center the display around the middle point

HDB12:   (HP-size/2)

HDE:     (size/2-1)|$400

# Registers

### VMODE Video/pixel mode	$F00028	RW

16        12         8         4         0

+---------+-------+-^----+----^--+----+-+

|  unused | width |cntrl |genlock|mode|e|

+---------+-------+------+-------+----+-+

15.....12 11...9  8....6 5.....3 2..1 0

enable (e):

turn on the video processing

mode:

0:   16-bit CrY

1:   24-bit RGB

2:   16-bit direct

3:   16-bit RGB

Choose the way the video chip should interpret and treat the linebuffer

data. Usually you'll set a '0' here for this nice CrY-mode.

The special "16-bit direct" mode needs a little explaining:

"each 32 bit entry is put in two-16bit words, which are

output to red and green on alternate phases of the

video clock...

It is assumed further processing will occur outside

the chip..." (spooky possums!)

Genlocking is not implemented in the Jaguar...

genlock:

bit 3    Genlock

bit 4    Incen()

bit 5    BINC()

control (cntrl):

bit 6    CSYNC   composite sync

bit 7    BGEN    background enable

bit 8    VarMOD  mixed mode pixels

You need to set CSYNC, period.

bit 7, when set, tells the video chip to clear the line buffer

to the colour in the background reg (BG). This has only effect

in the 16 bit linebuffer modes.

With bit 8, you can create mixed mode displays, partly consisting of

RGB pixels and partly of CrY pixels. The distinction is made by the

video chip with the least significant bit of the pixel. 1==CrY 0==RGB

You probably should set "mode" to 0 or 3 for "variable mode".

width:

bit 9-11 pixelwidth in video clock cycles-1

The display width should be set to an integer multiple of the pixel

width here

Use:  6C1   for 320x200 overscan 16bit Crymode

2C1   for 640x200 overscan 16bit Crymode

### BG  Background color	$F00058	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               pixelvalue              |

+---------------------------------------+

pixelvalue:

value to use to initalize the linebuffers in the videochip,

before the OP writes data into it.

### BORD Border color	$F0002A	WO

32      28       24       20      16      12       8       4       0

+-------^--------^--------^-------^-------^-------^-------^-------+

0 |                                 pixelvalue                      |

+-----------------------------------------------------------------+

This is the truecolor (32bit) pixel value that is used on those areas

of the screen, that aren't "covered" by the linebuffer. Basically the

Borders.. (read only)

### HP Horizontal Period / scanline length	$F0002E	WO

16        12         8         4         0

+---------^------+--^---------^---------+

|   unused       |         cycles       |

+----------------+----------------------+

9................0

cycles:

number of cycles in a half scanline

Defaults:   NTSC=844 PAL=850

### HBB Horizontal Border Begin	$F00030	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

cycle:

cycle number of the begin of the horizontal blank

Defaults:   NTSC=689+$400  PAL=687+$400

line: (l)

which halfline 0=left 1=rite

### HBE Horizontal Border End	$F00032	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

cycle:

cycle number of the end of the horizontal blank

Defaults:   NTSC=125  PAL=158

line: (l)

which halfline 0=left 1=rite

### HS HSYNC position	$F00034	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

cycle:

cycle number where the HSYNC signal should happen

Defaults:   NTSC=717+$400 PAL=725+$400

line: (l)

which halfline 0=left 1=rite

### HVS VSYNC Horizontal Position	$F00036	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

cycle:

cycle number where the VSYNC signal should happen

Defaults:   NTSC=651 PAL=601

line: (l)

which halfline 0=left 1=rite

### HDB1 Horizontal Display Begin #1	$F00038	WO

### HDB2 Horizontal Display Begin #2	$F0003A	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

line: (l)

which halfline 0=left 1=rite

cycle:

cycle number where the linebuffer display should begin for the first

linebuffer (HDB1) or the second linebuffer (HDB2). If you're only

using one linebuffer per scanline, set HDB2 to HDB1 or outside of the

value of HP.

Defaults:   NTSC=166 PAL=166

### HDE Horizontal Display End	$F0003C	WO

16        12         8         4         0

+---------^----+----^---------^---------+

|   unused   |l|         cycle          |

+--------------+------------------------+

10     9................0

cycle:

cycle number where the last linebuffer pixel should be

drawn

Defaults:   NTSC=672+$400 PAL=672+$400

line: (l)

which halfline 0=left 1=rite

### HEQ Horizontal Equalization Start	$F00054	WO

16        12         8         4         0

+---------^------+--^---------^---------+

|   unused       |         cycle        |

+----------------+----------------------+

10     9................0

cycle:

cycle number where the EQ signal should happen on both

scanlines (?)

Defaults:   NTSC=784 PAL=787

### VP Vertical Period / frame length	$F0003E	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanlines               |

+---------------------------------------+

scanlines:

number of scanlines in one full TV-refresh. Strangely the

Atari default values are (#scanlines - 2). Maybe you should put

in a value minus two here always...

Default: NTSC=523   PAL=623

### VBB Vertical Border Begin	$F00040	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

at this scanline vertical blanking should begin

Defaults:   NTSC=436 PAL=500

### VBE Vertical Border End	$F00042	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

at this scanline vertical blanking should end

Defaults:   NTSC=24 PAL=34

### VS VSYNC position	$F00044	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

at this scanline the VSYNC should start

Defaults:   NTSC=517 PAL=618

### VDB Vertical Display Begin	$F00046	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

at this scanline the display begins

Defaults:   NTSC=46 PAL=46

### VDE Vertical Display End	$F00048	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

at this scanline (inclusively) the display ends

Defaults:   NTSC=496 PAL=526

### VEB Vertical Equalization Start	$F0004A	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

begin of the equalization section

Defaults:   NTSC=511 PAL=600

### VEE Vertical Equalization End	$F0004C	WO

16        12         8         4         0

+---------^---------^---------^---------+

|               scanline                |

+---------------------------------------+

scanline:

end of the equalization section

Defaults: NTSC=6  PAL=6

### GENLOCK Genlock control	$F00026	W

32      28      24      20     16      12       8       4      0

+-------^-------^-------^-------^-------^-------^-------^------+

|                                                              | gen|

+---------------------------------------------------------+----+

genlocking (gen):

bit 0:   use external vblank source

bit 1:   unknown

bit 2:   use external hblank source

***Just a guess***  Try bit 2 at least once! Since the colors are a

bit off this might just something different still.

Writing a zero here will blank your screen. Careful! I don't know the

value (yet) to turn it on again.

### CLUT Color LookUp Table	$F00400	RW

Size: $400 bytes

Color lookup table for indirect pixelmodes (pixeldepth >= 8 pixels).

There are 256 slots of either 16 or 32 bit size. The size depends

on the selected linebuffer mode (see VMODE).

Apparently the CLUT in 16bit mode is mirrored at $F00600.

### HC Current index in linebuffer	$F00004	RW

16        12         8         4         0

+---------^-+--+----^---------^---------+

|  unused   |l2|          value         |

+-----------+--+------------------------+

10  9....................0

l2:      linebuffer which is active (#0 / #1)

value:   current index in the linebuffer

The horizontal counter, probably (but not definetely!) the counter of

the video chip, not the counter of the OP. I think there's really little

use for this. It's too late for on the fly PALlette changes now...

Very maybe you can se

### VC Current linebuffer number	$F00006	R

16        12         8         4         0

+---------^---------^---------^---------+

|                 value                 |

+---------------------------------------+

value:

Current scanline being processed by the OP.

### VI Video Interrupt control register	$F0004E	RW

16        12         8         4         0

+---------^---------^---------^---------+

|                 value                 |

+---------------------------------------+

When the VC register hits this value, then a GPU Level 0 interrupt

will be generated. This is used to generate a VBL for example. You

can use this also to get HBLANK IRQs, when you change the value

"on the fly". The VI value must be odd for non-interlaced displays.

### LPH Light pen/Laser Gun Horizontal coordinate	$F00008	R

16        12         8         4         0

+---------^---------^---------^---------+

|                 value                 |

+---------------------------------------+

Horizontal latched valued for a light gun. How does one interface

this thing ??

### LPV Light pen/Laser Gun Vertical coordinate	$F0000A	R

16        12         8         4         0

+---------^---------^---------^---------+

|                 value                 |

+---------------------------------------+

Vertical latched valued for a light gun.

### LBUFA Linebuffer A	$F00800	RW

### LBUFB Linebuffer B	$F01000	RW

Size: $800 bytes each

The actual addresses of the linebuffer.

Just reading concurrently from the linebuffers while the OP is

processing data produces glitches. Advice: Stay out of them!

### LBUFC Current linebuffer	$F01000	RW

Current linebuffer. The theory here is that the currently

active linebuffer is mapped to this address. Use carefully!

(Maybe with the GPU-Object of the OP ??)

# ADDENDUM

#### NTSC

Number of Scan Lines            525

Field Frequency                 59.94Hz

Scanline                        63.56us

##### PAL

Number of Scan Lines            625

Field Frequency                 50.0Hz

Scanline                        64 us